| Subject Code 22PC1AE209 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May /June 2025

APPLIED THERMODYNAMICS

(AE)

Time: 3 hours                                                                         Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define boiler efficiency and write down the expression for the same. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List at least two differences between jet condenser & surface condenser. | 2M | CO-1 | BL-2 |
|  | Define a steam nozzle and mention its major function. | 2M | CO-2 | BL-2 |
|  | Define volumetric efficiency of an air compressor and mention the effect of clearance volume on the volumetric efficiency. | 2M | CO-2 | BL-2 |
|  | Define dry bulb and wet bulb temperature as applied to psychrometry. | 2M | CO-3 | BL-1 |

PART-B				      5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Sketch and explain the working of Babcock and Wilcox Water-tube Boiler. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 2. | With neat sketches, explain (i) Parallel flow (ii) Counter flow type of jet condensers. | 10M | CO-2 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | Dry saturated steam enters a frictionless adiabatic nozzle with negligible velocity at a temperature of 300°C. It is expanded to a pressure of 5000 kPa. The mass flow rate is 1 kg/s. Calculate the exit velocity of steam. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 4. | Derive the condition for maximum efficiency of Parson’s reaction turbine. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | A single-stage single-acting air compressor delivers 0.6 kg of air per minute at 6 bar. The temperature and pressure at the end of suction stroke are 30°C and 1 bar. The bore and stroke of the compressor are 100 mm and 150 mm respectively. The clearance is 3% of the swept volume. Assuming the index of compression and expansion to be 1.3, find; (i) Volumetric efficiency of the compressor (ii) Power required if the mechanical efficiency is 85% | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 6. | Discuss the working of a Roots blower with a neat sketch and p-v diagram. | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | A gas turbine unit receives air at 1 bar and 300 K and compresses it adiabatically to 6.2 bar. The compressor efficiency is 88%. The fuel has a heating valve of 44186 kJ/kg and the fuel-air ratio is 0.017 kJ/kg of air. The turbine internal efficiency is 90%. Calculate the work of turbine and compressor per kg of air compressed and thermal efficiency. For products of combustion, cp = 1.147 kJ/kg K and γ = 1.333. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 8. | Compare solid and liquid propellant rockets. | 10M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | A simple vapour compression plant produces 5 tonnes of refrigeration The enthalpy values at inlet to compressor, at exit from the compressor, and at exit from the condenser are 183.19, 209.41 and 74.59 kJ/kg respectively Estimate (i) The refrigerant flow rate, rate (ii) The C O P (iii) The power required to drive the compressor, and (v) The rate of heat rejection to the condenser. Take 1 tonne = 14000 kJ/hr | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 10. | Explain (i) Specific humidity (ii) Degree of saturation (iii) Relative humidity. Also write down the expressions for the above in terms of partial pressures. | 10M | CO-3 | BL-3 |

***