| Subject Code 22PC1EE205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May /June 2025

CONTROL SYSTEMS

(Common to ECE and EIE)

Time: 3 hours                                                                         Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Derive the transfer function series RLC Electrical network | 2M | CO-3 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define rise time and peak time. | 2M | CO-1 | BL-1 |
|  | State the Nyquist Stability criterion. | 2M | CO-1 | BL-1 |
|  | What are the advantages of PID controllers? | 2M | CO-4 | BL-2 |
|  | Write the conditions for a system to be controllable and observable. | 2M | CO-3 | BL-2 |

PART-B				       5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Derive from fundamentals, the transfer function of AC Servo motor. | 5M | CO-3 | BL-2 |
|  | b) Explain the following terms related to signal flow graph: (i) Node (ii) Branch (iii) Forward path gain (iv) Loop gain | 5M | CO-3 | BL-1 |
| OR |  |  |  |  |
| 2. | Find the transfer function X(s)/F(s) of the following system | 10M | CO-3 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - What are the necessary conditions to have all the roots of the characteristic equation in the left half of s-plane? | 5M | CO-1 | BL-2 |
|  | - Explain the time - response specifications of a standard second order system. | 5M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 4. | - For a unity back system with open loop transfer function is given by, Gs=10S(S+2) Determine damping ratio, Natural undamped resonance frequency, Percentage peak over shoot for a unit step input function. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - A feedback system has an open-loop transfer function GsHs=Kss+3(s2+2s+2) Find the root locus as K varied from 0 to infinity. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 6. | Draw Bode plot for. Determine gain and phase crossover frequencies. | 10M | CO-1 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Derive the expression for the transfer function of a PID controller. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | Derive the transfer function, electrical network realization of lead compensation and also draw the frequency response. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | a) Find the transfer function of the system having the state model, x1x2=01-3-4x1x2+01u(t) yt=10x1x2 | 5M | CO-3 | BL-3 |
|  | b) State and prove properties of state transition matrix. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 10. | Obtain the state space representation of the following transfer function using controllable canonical form Y(s)U(s)=2s2+3s+1s3+5s2+6s+7 | 10M | CO-3 | BL-3 |

***