| Subject Code 22PC1AE208 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May/June 2025

FLUID MECHANICS AND HEAT TRANSFER

(AE)

Time: 3 hours                                                                                Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Note: Heat Transfer Data Book is allowed.

PART-A					5X2=10M

|  | State Newton’s law of Viscosity | 2M | CO1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Discover forces acting on the fluid in motion | 2M | CO2 | BL-2 |
|  | State Fourier heat conduction. | 2M | CO3 | BL-3 |
|  | Define the thermal boundary layer. | 2M | CO4 | BL-1 |
|  | Differentiate parallel and counterflow heat exchanger. | 2M | CO4 | BL-3 |

PART-B				     5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Derive Continuity equation. And state assumptions. | 7M | CO1 | BL-1 |
|  | - Discuss the types of fluid flows. | 3M | CO1 | BL-2 |
| OR |  |  |  |  |
| 2. | a) A mercury manometer is connected to a horizontal 20°C water pipe flow at two points. If the manometer reading is h = 55 cm, what is the pressure drop between the two points? b) What is a Manometer? How are they classified? | 6M 4M | CO1 CO1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |
| 3. | a) Water at a head of 300m is supplied to a pipe of diameter 40 m and 4m length. Calculate power transmitted or power required to maintain the flow for 2m/s and friction factor f= 0.009. | 5M | CO2 | BL-3 |
|  | b) List the various minor energy losses in flow through pipes with their expressions. | 5M | CO2 | BL-2 |
| OR |  |  |  |  |
| 4. | Derive Bernoulli’s equation from basic principles by stating the assumptions. | 10M | CO2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | a) Calculate the rate of heat loss Q through a wall of red brick (k= 0.7W/m K) 5m in length, 4m in height, and 250mm in thickness if the wall surfaces are maintained at 373K and 303K, respectively. | 5M | CO3 | BL-3 |
|  | b) What is thermal contact resistance? How does it affect heat transfer? | 5M | CO3 | BL-2 |
| OR |  |  |  |  |
| 6. | Derive an expression for 3-dimentional cartesian heat conduction equation. | 10M | CO3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | a) When 0.03m/sec of velocity is passed through a tube of 2 cm diameter, it is found to be heated from 20oC to 60oC. The heating is achieved by condensing steam on the surface of the tube, and subsequently, the surface temperature of the tube is maintained at 90oC. Determine the length of the tube required for a fully developed. Flow. | 5M | CO3 | BL-4 |
|  | b) List the non-dimensional numbers used in convection. And explain their significance. | 5M | CO3 | BL-3 |
| OR |  |  |  |  |
| 8. | Determine the heat transfer rate by free convection from a plate 0.3m×0.3m for which one surface is insulated and the other surface is maintained at 110 oC and exposed to atmosphere air at 300 C when the plate is vertical. | 10M | CO3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | a) State and explain the Planck Law and Stefan Boltzmann law of radiation. | 5M | CO4 | BL-2 |
|  | b) Why houses are painted white in hot country? | 5M | CO4 | BL-4 |
| OR |  |  |  |  |
| 10. | A counterflow heat exchanger has an overall heat transfer coefficient of 225W/m2K and a surface area of 33 m2. The hot fluid [Cp=3.56kJ/kgK] enters at 940C and flows at 2.52kg/s. The cold fluid [Cp=1.67kJ/kgK] enters at 160C and flows at 2.27kg/s. Determine the rate of heat transfer. | 10M | CO4 | BL-4 |

***