| Subject Code 22PC1ME207 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May/June 2025

FLUID MECHANICS AND HYDRAULIC MACHINES

(ME)

Time: 3 hours                                                                                 	   Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | State viscosity and write the types of viscosity | 2M | CO-1,2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Name the assumption of Bernoulli’s theorem. | 2M | CO-2,3 | BL-1 |
|  | Define the terms work done and efficiency of jet? | 2M | CO-3,1 | BL-1 |
|  | State the function of draft tube and its types. | 2M | CO-4,3 | BL-1 |
|  | Define the terms, Slip and Negative slip in reciprocating pumps | 2M | CO-4,3 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - A 400 mm diameter shaft is rotating at 200 rpm in a bearing of length 120 mm. if the thickness of oil film is 1.5 mm and dynamic viscosity of the oil is 0.7 Ns/m2, determine (i) torque required to overcome friction in bearing (ii) power utilization in overcoming viscous resistance. | 5M | CO-1,2 | BL-3 |
|  | - What is the difference between U-tube differential manometer and inverted U-tub e differential manometer? Where are they used? | 5M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
| 2. | The velocity components in a two-dimensional incompressible flow field are expressed as u= y33+2x- x2y v=xy2-2y- x33 - Determine velocity and acceleration at point (1, 3). - is the flow physically possible? If so obtain an expression for the steam function. - is the flow irrotational? If so obtain an expression for the velocity potential function. | 10M | CO-1,2 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Derive the Bernoulli’s equation of motion along a stream line. | 5M | CO-2,3 | BL-2 |
|  | - An orifice plate of diameter 10 cm has been fitted into a 25 cm diameter pipe that conveys oil of specific gravity 0.8. The pressure differential on the two sides of the orifice plate is measured by a mercury oil differential manometer. If the gauge shows a deflection of 80 cm of mercury, calculate the oil discharge. Take coefficient of discharge Cd = 0.65. | 5M | CO-2,3 | BL-3 |
| OR |  |  |  |  |
| 4. | - Find head loss due to friction in a pipe of diameter 300 mm and length 50 m, through which water is flowing at a velocity of 3 m/s. Take f=0.1. | 5M | CO-2,3 | BL-2 |
|  | - Obtain an expression for discharge through a venture meter. | 5M | CO-2,3 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | - Derive an expression for the force exerted by a jet striking the stationary curved plate at centre, when the plate is symmetrical. | 5M | CO-2,3 | BL-2 |
|  | - A 15 cm diameter jet of water with a velocity of 15 m/s strikes a plane normally. If the plate is moving with a velocity of 6 m/s in the direction of the jet calculate the work done per second on the plate and the efficiency of jet. | 5M | CO-2,3 | BL-3 |
| OR |  |  |  |  |
| 6. | A jet of water having a velocity of 35 m/s impinges on a curved vane moving with a velocity of 20 m/s. the jet makes an angle of 30° to the direction of motion of vanes when entering and leaves at an angle of 120°. Draw the triangles of velocities at inlet and outlet and find (i) the angle of vane tips so that water enters and leaves without shock (ii) the work done per N of water entering the vanes and (iii) the efficiency. | 10M | CO-3,4 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Classify hydraulic turbines in detail. | 5M | CO-3,4 | BL-2 |
|  | b) A Pelton wheel has to be designed for the following data. Power to be developed = 6000 kW. Net head available = 300 m; Speed = 550 r.p.m.; Ratio of jet diameter to wheel diameter = 1/10 and overall efficiency = 85%. Find the number of jets, diameter of the jet, diameter of the wheel and the quantity of water required. Take Kv=0.985 and Ku=0.45. | 5M | CO-3,4 | BL-3 |
| OR |  |  |  |  |
| 8. | a) Explain the elements of hydraulic power plant with neat sketch. | 5M | CO-1,4 | BL-2 |
|  | b) A turbine develops 7725 kW under a head of 28 m at 140 rpm. Calculate the specific speed of the turbine and state the type of turbine | 5M | CO-1,4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | a) Explain the working of a single-stage centrifugal pump with neat sketch. | 5M | CO-1,4 | BL-2 |
|  | b) Derive an expression for the specific speed of a centrifugal pump. | 5M | CO-3,4 | BL-3 |
| OR |  |  |  |  |
| 10. | A single acting reciprocating pump running at 60 rpm delivers 0.53 m3 of water per minute. The diameter of the piston is 200 mm and stroke length 300 mm. The suction and delivery heads are 4 m and 12 m respectively. Determine: - Theoretical discharge. - Coefficient of discharge. - Percentage slip of the pump. - Power required to run the pump. | 10M | CO-3,4 | BL-3 |

***