| Subject Code 22PC1CE206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May/June 2025

HYDRAULIC ENGINEERING AND HYDRAULIC MACHINES

(CE)

Time: 3 hours                                                                                 		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Describe the most economical section of the channel. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Differentiate between gradually varied flow and rapidly varied flow. | 2M | CO-1 | BL-2 |
|  | Explain the importance of dimensional homogeneity in dimensional analysis. | 2M | CO-2 | BL-2 |
|  | Explain the difference between an impulse turbine and a reaction turbine. | 2M | CO-3 | BL-2 |
|  | Define cavitation and its effects in pumps. | 2M | CO-4 | BL-2 |

PART-B		  		         5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the conditions for the most economical rectangular channel section. | 5M | CO-1 | BL-2 |  |
|  | - Calculate the most economical section of a rectangular channel carrying water at the rate of 0.5 m3/sec, the bed slope of the channel being 1 in 2000. Chezy’s constant (C) is 50. | 5M | CO-1 | BL-3 |  |
| OR |  |  |  |  |
| 2. | A channel of the trapezoidal section has a bed width of 8 m and a bed slope of 1 in 4000. If the flow depth is 2.4 m and the side slopes of the channel are 1 horizontal to 3 vertical, calculate the mean velocity of flow and discharge. Chezy’s constant (C) is 55. | 10M | CO-1 | BL-3 |  |
| UNIT-II |  |  |  |  |
| 3. | - Derive the dynamic equation of gradually varied flow. | 5M | CO-1 | BL-2 |  |
|  | b) A sluice gate discharges water into the horizontal rectangular channel with a velocity of 10 m/sec and a depth of flow of 1 m. Calculate the depth of flow of water after the jump and the consequent loss in total head. | 5M | CO-1 | BL-3 |  |
| OR |  |  |  |  |
| 4. | In a rectangular channel of 0.5 m width, a hydraulic jump occurs at a point where the depth of flow is 0.15 m and the Froude number is 2.5. Calculate (i) the specific energy, (ii) the critical and subsequent depths, (iii) the loss of head. | 10M | CO-1 | BL-3 |  |
| UNIT-III |  |  |  |  |
| 5. | - Calculate the dimensions of the following quantities: (i) Discharge, (ii) Dynamic viscosity, (iii) Kinematic viscosity, (iv) Force, and (v) Specific weight | 5M | CO-2 | BL-3 |  |
|  | - Explain any two dimensionless numbers and their significance. | 5M | CO-2 | BL-2 |  |
| OR |  |  |  |  |
| 6. | An oil of specific gravity 0.92 and viscosity 0.03 poise is to be transported at the rate of 2500 litres/sec through a 1.2 m diameter pipe. Tests were conducted on a 12 cm diameter pipe using water at 200 °C. If the viscosity of water at 200C is 0.01 poise, calculate: (i) Velocity of flow in the model, and (ii) Rate of flow in the model. | 10M | CO-2 | BL-3 |  |
| UNIT-IV |  |  |  |  |
| 7. | - Describe different types of draft tubes. Why is it used in a reaction turbine? | 5M | CO-3 | BL-2 |  |
|  | - Explain the elements of a typical Reaction turbine. | 5M | CO-3 | BL-2 |  |
| OR |  |  |  |  |
| 8. | A pelton wheel has a mean bucket speed of 10 meters per second with a jet of water flowing at the rate of 700 litres/s under a head of 30 metres. The buckets defect the jet through an angle of 1600. Calculate the power given by water to the runner and the hydraulic efficiency of the turbine. Assume co-efficient of velocity as 0.98. | 10M | CO-3 | BL-3 |  |
| UNIT-V |  |  |  |  |
| 9. | - Explain the different types of casings of a centrifugal pump. | 5M | CO-4 | BL-2 |  |
|  | - Explain the classification of Hydropower plants. | 5M | CO-4 | BL-2 |  |
| OR |  |  |  |  |
| 10. | The diameters of the impeller of a centrifugal pump at the inlet and outlet are 300 mm and 600 mm, respectively. The velocity of flow at the outlet is 2.5 m/sec, and vanes are set back at an angle of 450 at the outlet. Calculate the minimum starting speed of the pump if the manometric efficiency is 75%. | 10M | CO-3 | BL-3 |  |

***