| Subject Code 22PC1EI204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May/June 2025

LINEAR IC APPLICATIONS

(EIE)

Time: 3 hours                                                                                 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Assume that an op-amp has input bias currents IB1=400nA and IB2=300nA.Determine the average bias current and the offset current. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Draw the circuit diagram of voltage to current converter with floating load. | 2M | CO-2 | BL-2 |
|  | What are the characteristics of a comparator? | 2M | CO-2 | BL-1 |
|  | Define Capture range and Lock range of PLL. | 2M | CO-3 | BL-2 |
|  | The basic step of a 9-bit DAC is 10.3mV.If 000000000 represents 0V, what output is produced if the input is 101101100? | 2M | CO-4 | BL-3 |

PART-B				       5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Draw and Explain the circuit diagram of Dual input balanced output differential amplifier. Also derive the expression for deferential gain. | 10M | CO-1 | BL-2 |  |
| OR |  |  |  |  |
| 2. | - Define any five DC characteristics of op-amp. | 5M | CO-1 | BL-1 |  |
|  | - Draw the equivalent circuit of an op-amp. Also mention its ideal characteristics. | 5M | CO-1 | BL-2 |  |
| UNIT-II |  |  |  |  |
| 3. | - What are the different types of op-amp configurations? Explain any one of them. | 5M | CO-1 | BL-2 |  |
|  | - Draw and explain the circuit of practical integrator with frequency response. | 5M | CO-2 | BL-2 |  |
| OR |  |  |  |  |
| 4. | What are the important features of instrumentation amplifier? Derive the expression for output voltage with the help of differential amplifier. | 10M | CO-2 | BL-3 |  |
| UNIT-III |  |  |  |  |
| 5. | - Draw the circuit diagram of non-inverting comparator. Explain its operation with the help of input and output waveforms. | 5M | CO-2 | BL-3 |  |
|  | - Design a square wave generator to operate at a frequency of 2KHz. | 5M | CO-2 | BL-4 |  |
| OR |  |  |  |  |
| 6. | - Draw the basic circuit of first order high pass Butterworth filter and derive the expression for transfer function. | 5M | CO-2 | BL-2 |  |
|  | - Draw the circuit all pass filter. Explain its operation with necessary expressions and wave forms. | 5M | CO-2 | BL-2 |  |
| UNIT-IV |  |  |  |  |
| 7. | - Draw the block diagram of phase locked loop. Explain in detail. | 5M | CO-3 | BL-2 |  |
|  | - What are the advantages and disadvantages of monolithic PLLs over discrete PLLs? | 5M | CO-3 | BL-1 |  |
| OR |  |  |  |  |
| 8. | Draw the functional block diagram of IC555 timer as a monostable multivibrator. Explain its operation. | 10M | CO-3 | BL-2 |  |
| UNIT-V |  |  |  |  |
| 9. | - Draw the circuit diagram of weighted resistor DAC. Explain its operation. | 5M | CO-4 | BL-2 |  |
|  | - Define the following specifications of DAC/ADC (i) Resolution (ii) Linearity (iii) Accuracy (iv) Monotonicity (v) Settling Time | 5M | CO-4 | BL-1 |  |
| OR |  |  |  |  |
| 10. | Explain the operation of successive approximation ADC with neat sketch. Mention its advantages and disadvantages. | 10M | CO-4 | BL-2 |  |

***