| Subject Code 22PC1EE204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular /Supplementary Examinations, May/June 2025

POWER SYSTEMS - I

(EEE)

Time: 3 hours                                                                                 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Give two reasons why Nuclear Power Plants are located away from populated areas? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain Ferranti effect? | 2M | CO-2 | BL-2 |
|  | Mention the factors affecting corona? | 2M | CO-3 | BL-2 |
|  | Mention the types of insulators and define string efficiency? | 2M | CO-3 | BL-2 |
|  | Explain what is meant by a radial distribution system. | 2M | CO-4 | BL-2 |

PART-B				       5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Draw and label the layout of a Thermal Power Plant. Briefly describe its main components. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | With a labeled diagram, explain the working of a Hydroelectric Power Plant. Also explain the function of each component. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | a) Show that the inductance per unit length of an overhead line due to internal flux linkages is constant and is independent of size of conductor. | 5M | CO-2 | BL-2 |
|  | b) Explain about the ‘skin effect’ and ‘proximity effect’ when referred to overhead lines. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | a) Draw the phasor diagram of a short transmission line and derive an expression for voltage regulation. | 5M | CO-2 | BL-2 |
|  | b) An overhead, single-phase transmission line delivers 1100 kW at 33 kV at 0.8 p.f. lagging. The total resistance of the line is 10 Ω and total inductive reactance is 15 Ω. Determine i) sending-end voltage ii) transmission efficiency | 5M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Describe Phenomenon of corona. | 5M | CO-3 | BL-3 |
|  | b) Determine the critical disruptive voltage for a 3-phase line operating at 110 kV which has conductor of 1.25 cm diameter arranged in delta with spacing of a 3.05 m. Assume air density factor of 1.07 and the dielectric strength of air to be 21 kV/cm. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Determine the Reflection and Refraction Coefficients for a transmission line with Z as Surge Impedance when receiving end is terminated by (i) Open circuit and (ii) Short Circuit | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Derive the expressions for sag and tension when the supports are at unequal heights. | 5M | CO-3 | BL-2 |
|  | b) A transmission line has a span of 150 m between level supports. The conductor has a cross-sectional area of 2 cm2. The tension in the conductor is 2000 kg. If the specific gravity of the conductor material is 9·9 gm/cm3 and wind pressure is 1·5 kg/m length, calculate the sag. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | Explain about the Potential Distribution over Suspension Insulator String and also discuss about methods to improve the string efficiency across suspension type insulators. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | a) A 2-wire d.c. distributor cable AB is 2 km long and supplies loads of 100A,150A,200A and 50A situated 500m, 1000m, 1600m and 2000m from the feeding point A. Each conductor has a resistance of 0·01 Ω per 1000 m. Calculate the p.d. at each load point if a p.d. of 300 V is maintained at point A. | 5M | CO-4 | BL-3 |
|  | b) Compare the DC Distribution and AC Distribution Systems? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Determine the voltage drop in a uniformly loaded D.C distributor fed at both ends with (i) equal voltages (ii) unequal voltages. | 10M | CO-4 | BL-2 |

***