| Subject Code 22PC1IN201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May /June 2025

SENSORS AND DEVICES

(CSE-IoT)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What are the major classifications of sensors? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain the principle of a single-wire thermometer. | 2M | CO-2 | BL-2 |
|  | Define Signal Conditioning. | 2M | CO-3 | BL-2 |
|  | Compare sensor and smart sensor. | 2M | CO-4 | BL-4 |
|  | What is the purpose of a serial interface in IoT hardware? | 2M | CO-5 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Explain the classification of sensors based on their operating principles and provide examples for each category. | 5M | CO-1 | BL-2 |
|  | - Explain the static characteristics of sensors. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | Discuss the key performance characteristics of sensors. How do these characteristics influence sensor selection for specific applications? | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | - Discuss the procedure for interfacing DHT11 sensor. | 5M | CO-2 | BL-2 |
|  | - Discuss the techniques used for sensor interfacing, including Port Programming. ADC, and SPi. Highlight the advantages and limitations of each technique. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4. | Explain the significance of PWM generation and demodulation in sensor interfacing with suitable signal flow across the design, Also provide a practical example illustrating its application in electronic systems. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5. | - Explain the principle of operation and working of linearization of resistive bridge sensor. | 5M | CO-3 | BL-3 |
|  | - Describe the Components Used in Data Acquisition Systems. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6. | Illustrate the objectives and configuration of Data Acquisition System. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Explain the architecture of a smart sensor, including its internal components. | 5M | CO-4 | BL-2 |
|  | - Explain different error compensation techniques. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8. | Elaborate on the process of information coding and processing in smart sensors. How does this contribute to enhancing their functionality? | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | - Compare of Serial, SPI, and I2C interfaces in IoT. | 5M | CO-5 | BL-4 |
|  | - Briefly discuss about the peripherals of Raspberry Pi. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Walk through the steps involved in connecting and controlling servomotor using Arduino. Provide a basic code example. | 10M | CO-5 | BL-4 |

***