| Subject Code 22PC1CE204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May/June 2025

STRENGTH OF MATERIALS-II

(CE)

Time: 3 hours                                                                                 		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define principal stresses and principal planes. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the assumptions in the theory of Pure Torsion? | 2M | CO-2 | BL-2 |
|  | Differentiate thick and thin cylinders. | 2M | CO-3 | BL-1 |
|  | Define Euler’s Theory. | 2M | CO-4 | BL-1 |
|  | Define eccentric loading. | 2M | CO-4 | BL-1 |

PART-B				      5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | At a point in a strained material the principal tensile stresses across two perpendicular planes are 80N/mm2 and 40 N/mm2. Determine normal stress, shear stress and the resultant stress on a plane inclined at 200 with the major principal plane. Determine also the obliquity. | 10M | CO-1 | BL-3 |  |
| OR |  |  |  |  |
| 2. | An element in a stressed material has tensile stress of 500 N/mm2 and a compressive stress of 350 N/mm2 acting on two mutually perpendicular planes and equal shear stresses of 100 N/mm2 on these planes. Find the Principal stresses and the position of the principal planes. Determine the maximum shearing stress also. | 10M | CO-1 | BL-3 |  |
| UNIT-II |  |  |  |  |
| 3. | A hollow shaft having an inside diameter 70% of its outside diameter is to replace a solid shaft transmitting the same power at the same speed. Calculate the percentage aving in material, if material to be used is the same. | 10M | CO-2 | BL-3 |  |
| OR |  |  |  |  |
| 4. | A closely coiled helical spring of round steel wire 10 mm in diameter having 10 complete turns with a mean diameter of 120 mm is subjected to an axial load of 200 N. Determine i) The deflection of the spring ii) Maximum shear stress in the wire iii) Stiffness of the spring. Take (C)=8x104 N/mm2. | 10M | CO-2 | BL-3 |  |
| UNIT-III |  |  |  |  |
| 5. | Find the thickness of metal necessary for a cylindrical shell of internal diameter 160mm to withstand an internal pressure of 8 N/mm2. The maximum hoop stress in the section is not to exceed 35 N/mm2. | 10M | CO-3 | BL-3 |  |
| OR |  |  |  |  |
| 6. | A shell 3.25 meters long, 1 meter in diameter is subjected to an internal pressure of 1 N/mm2. If the thickness of the shell is 10mm, find the circumferential and longitudinal stresses. Find also the maximum shear stress and the changes in the dimensions of the shell. Take E=2 X 105 N/mm2 and 1/m =0.3 | 10M | CO-3 | BL-3 |  |
| UNIT-IV |  |  |  |  |
| 7. | - Derive Euler’s formula for the critical load of a column with both ends hinged. | 6M | CO-4 | BL-2 |  |
|  | - Find the ratio of the strength of a solid circular column with that of a hollow circular column of equal area, whose internal diameter is 2/3 the external diameter. Both the columns are the same material, having the same length and are hinged at their ends. | 4M | CO-4 | BL-3 |  |
| OR |  |  |  |  |
| 8. | Compare the crippling loads given by Euler’s and Rankine’s formula for a tubular steel strut of outer diameter 40mm and wall thickness 4 mm. Length of the strut is 2.5 m. fc=32 N/mm2, =1/7500, E=2.1 x 105 N/mm2. The strut is fixed at both ends. | 10M | CO-4 | BL-3 |  |
| UNIT-V |  |  |  |  |
| 9. | A rectangular column of width 200 mm and depth of 150mm carries a point load of 240 kN at an eccentricity of 10 mm. Determine the maximum and minimum stresses on the section. | 10M | CO-4 | BL-3 |  |
| OR |  |  |  |  |
| 10. | Derive the expressions for maximum principal stress theory and maximum shear stress theory. | 10M | CO-4 | BL-2 |  |

***