| Subject Code 22PC1CE205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular /Supplementary Examinations, May/June 2025

STRUCTURAL ANALYSIS

(CE)

Time: 3 hours                                                                                 			Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define Perfect truss | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Mohr’s Theorem. | 2M | CO-2 | BL-1 |
|  | Define propped beam. | 2M | CO-3 | BL-1 |
|  | Write the Theorem of three moments with usual notations. | 2M | CO-3 | BL-1 |
|  | Define ILD. | 2M | CO-4 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Find the magnitude and nature of forces in all the members of truss shown in fig. by method of joints. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Calculate the forces in all the members of the truss shown below by the method of Sections | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | A rolled steel beam (simply supported) having a span of 6 m carries a point load of 40 kN at a distance of 4 m from left support. Find the deflection under the load and also find the position and amount of maximum deflection. Take E = 200 kN/mm2; I = 156·25 × 106 mm4. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | A cantilever 4 m span carries two point loads of each 20 kN at 0 m and 2 m from free end. Find the max. slope and deflection using the moment area method. Take EI 8400x 10 9 N-mm2 . | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | A cantilever of 8m length carries an U.D.L of 10 kN/m over the full span and a central concentrated load of 10 kN. If the free end is supported by a prop, find the reaction at the prop and also draw the S.F. and B.M. diagrams | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Find fixed end moments for the fixed beam shown in below figure | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Analyze the continuous beam shown in figure, using three-moment equation. Draw S.F and B. M diagrams | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8. | Analyse a three span continuous beam carrying udl of wkN/m throughout the three spans by Clayperon’s theorem. Draw BMD | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | Draw the Influence line diagram for reactions of a simply supported beam of 12 m span. Also draw the influence line diagrams for Shear force and bending moments at quarter span and mid-span sections | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10. | Draw the influence line for BM and SF for a section 8 m from the left hand support A shown in figure below. Determine the maximum BM and SF values for simply supported span 28 m. The section carries a uniformly rolling load 5 kN/m over a span of 9 m. ‘ | 10M | CO-4 | BL-4 |

***