| Subject Code 22PC1AE207 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular /Supplementary Examinations, May/June 2025

THEORY OF MACHINES AND MECHANISMS

(AE)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Write Grubler criterion to the find the degrees of freedom for planar mechanisms. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Coriolis acceleration. When does it occur in the motion of a link in a machine | 2M | CO-1 | BL-1 |
|  | Define a cam and a follower. State two practical uses of cam mechanisms in machines. | 2M | CO-2 | BL-1 |
|  | How does spur gear is different from helical gear. | 2M | CO-3 | BL-1 |
|  | What is hunting of governor? | 2M | CO-4 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Explain the difference between mechanism and machine. | 5M | CO-1 | BL-2 |
|  | - What is inversion? Explain with help of suitable sketches the inversion of slider crank chain mechanism. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | a) Explain degrees of freedom and it’s the significance of a kinematic chain when it functions as a mechanism? Give examples. | 5M | CO-1 | BL-2 |
|  | b) For the kinematic linkages shown in figure (a) and (b). Identify the number of binary links, quaternary links, total number of links and degrees of freedom. | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | PQRS is a four bar chain with link PS fixed. The lengths of the links are PQ= 62.5 mm ; QR = 175 mm ; RS = 112.5 mm ; and PS = 200 mm. The crank PQ rotates at 10 rad/s clockwise. Draw the velocity and acceleration diagram when angle QPS = 60° and Q and R lie on the same side of PS. Find the angular velocity and angular acceleration of links QR and RS. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 4. | The lengths of crank OB and connecting rod AB are 100 mm and 400 mm respectively. If the crank rotates clockwise with an angular velocity of 10 rad/s, find: 1.Velocity of the slider A, and 2.Angular velocity of the connecting rod AB. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | A cam drives a Knife edge reciprocating follower in the following manner: During first 120° rotation of the cam, follower moves outwards through a distance of 20 mm with simple harmonic motion. The follower dwells during next 30° of cam rotation. During next 120° of cam rotation, the follower moves inwards with simple harmonic motion. The follower dwells for the next 90° of cam rotation. The minimum radius of the cam is 25 mm. Draw the profile of the cam. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 6. | An aeroplane makes a complete half circle of 50 metres radius, towards left, when flying at 200 km per hr. The rotary engine and the propeller of the plane has a mass of 400 kg and a radius of gyration of 0.3 m. The engine rotates at 2400 r.p.m. clockwise when viewed from the rear. Find the gyroscopic couple on the aircraft and state its effect on it. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Mention the important types of gears and discuss their applications | 5M | CO-3 | BL-2 |
|  | - Explain briefly the differences between simple, compound, and epi-cyclic gear trains. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 8. | In a reverted epicyclic gear train, the arm A carries two gears B and C and a compound gear D - E. The gear B meshes with gear E and the gear C meshes with gear D. The number of teeth on gears B, C and D are 75, 30 and 90 respectively. Find the speed and direction of gear C when gear B is fixed and the arm A makes 100 r.p.m. clockwise. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | Each arm of a Porter governor is 250 mm long and is pivoted to the axis of rotation. Each ball has a mass of 5 kg and the sleeve weight is 20 kg. The radius of rotation of the ball is 200 mm when the governor begins to lift and 250 mm when its speed is maximum. Determine the maximum and minimum speeds and the range of speed of the governor. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 10. | For a spring controlled Hartnell type governor, following data is provided: Mass of the governor ball = 1.80 kg Length of the vertical bell crank lever = 8.75 cm Length of other arm of bell crank lever = 10.0 cm The speeds corresponding to radii of rotations of 12 cm and 13 cm are 296 and 304 r.p.m. respectively. Find the stiffness of spring. | 10M | CO-4 | BL-3 |

***