| Subject Code 22PC1CE207 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular/Supplementary Examinations, May /June 2025

TRANSPORTATION ENGINEERING

(CE)

Time: 3 hours                                                        		        Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | List the recommendations of Jayakar committee. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define stopping sight distance and list the factors affecting it. | 2M | CO-2 | BL-1 |
|  | Name any four tests conducted on aggregates for pavement construction. | 2M | CO-3 | BL-1 |
|  | What is coning of wheels? State its importance. | 2M | CO-4 | BL-1 |
|  | What is grade compensation and why is it required on curves? | 2M | CO-4 | BL-1 |

PART-B				       5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - What is highway alignment? Explain in detail the basic requirements of a highway alignment. | 5M | CO-1 | BL-2 |
|  | - Discuss in detail the Nagpur and Bombay Road Development Plans. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Explain various engineering surveys required for selecting highway location. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Explain the factors controlling the geometric design of highways. | 5M | CO-2 | BL-2 |
|  | - Compute the Stopping Sight Distance (SSD) for a vehicle moving at 60 kmph on a level road. Assume reaction time = 2.5 sec, and coefficient of friction = 0.35. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Design the super elevation and extra widening for a horizontal curve on a two-lane highway. Given: Design speed = 80 kmph Radius of the curve = 250 m Width of the carriageway = 7.0 m Wheelbase of the vehicle = 6.1 m Lateral friction factor (f) = 0.15 Maximum permissible super elevation = 7% Also, determine whether the calculated super elevation is within the permissible limit. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | Illustrate different components of flexible and rigid pavements with neat sketches. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | Explain any four tests conducted to check the quality of bitumen used in pavement construction. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | For a B.G. railway curve of 300 m radius, vehicle wheelbase = 6.5 m, flange clearance = 1.5 mm, diameter = 1.5 m, calculate extra widening. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | Explain the components of Permanent Way in detail. Discuss the functions and specifications of rails, sleepers, ballast, and subgrade. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | A 60 curve branches of from a 30 main curve in an opposite direction in the layout of a BG rail. If the speed of the branch line is restricted to 55 kmph. Determine the restricted speed on main line. Assuming the permissible cant deficiency as 75mm. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10. | Illustrate various components of turnout with a neat sketch. | 10M | CO-4 | BL-2 |

***