| Subject Code 22PC1EE205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

CONTROL SYSTEMS

(EEE)

Time: 3 hours                                                                       		 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | What are the advantages of negative feedback? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | The overall transfer function of a certain system is K/(s+K). The system requires 0.9 second to indicate 95% of response when subjected to a unit step excitation. Calculate the value of K. | 2M | CO-2 | BL-4 |
|  | What is gain margin and phase margin? | 2M | CO-4 | BL-1 |
|  | Differentiate between a PI and PID controller. | 2M | CO-4 | BL-2 |
|  | If φ(t) is the state transition matrix, then determine φ (t). | 2M | CO-3 | BL-3 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Derive the transfer function of a DC motor . | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 2. | Draw the signal flow graph for the given block diagram and obtain the transfer function by using Mason’s Gain formula | 10M | CO-2 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | - The characteristic equation of a system is given by s5+4s4+5s3+10s2+6s+10=0. Is the system stable? Justify. | 5M | CO-3 | BL-3 |
|  | - For a unity feedback system, Gs=(s+20)(s+50)(s+1) Find the steady state error for a unit step input. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 4. | For a unity feedback system having open loop transfer function as Gs= (s+2)s2(s2+7s+12) , determine static error constants Kp, Kv, and Ka and also steady state error for unit step input. | 10M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Sketch the root locus for the open loop transfer function of unity feedback control system Gs= Ks(s+1)(s+10). | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Draw the Bode plot of GS=20S(S+1)(10S+5) and determine the gain and phase cross over frequencies. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Draw the circuit of lag and lead compensators and derive their transfer functions | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | What is a lead compensator? When is this compensation technique employed? Discuss in detail the procedure for design of lead compensator using bode plot. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | Comment on the stability, controllability and observability of a system with A=1568 B=01 C=11. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 10. | The transfer function of a SISO system is GS=10S4+3S3+5S2+2S+2. Find the state space representation | 10M | CO-2 | BL-3 |

***