| Subject Code 22PC1IN202 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

COMPUTER NETWORKS

(Common to CSE, IT and AIML)

Time: 3 hours                                                                      		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | How does the OSI layered structure facilitate communication in networks? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain the functions of Data link layer. | 2M | CO-2 | BL-2 |
|  | Relate IPV4 and IPV6? | 2M | CO-3 | BL-2 |
|  | Describe how the Token Bucket algorithm controls network traffic. | 2M | CO-4 | BL-2 |
|  | What is the primary function of a firewall in network security? | 2M | CO-5 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Describe the OSI model and its layers. | 5M | CO-1 | BL-2 |
|  | - Compare and contrast different network topologies. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | What is Frequency Division Multiplexing, and how does it differ from other multiplexing techniques? Ten signals, each requiring 4000Hz, are multiplexed into a single channel using FDM. What is the minimum bandwidth required for the multiplexed channel? Hint: Assume that the guard band are 400Hz wide. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | - What is the need of flow control? Explain the common approaches for flow control in data link layer. | 6M | CO-2 | BL-2 |
|  | - Explain the CRC error detection technique with an example. | 4M | CO-2 | BL-2 |
| # OR |  |  |  |  |  |
| 4. | Find the minimum Hamming distance for Correction of 2 errors and Detection of 3 errors or correction of 2 errors. A block of bits with n rows and k columns uses horizontal and vertical parity bits for error detection. Derive an expression for the probability that the error will be undetected. Hint: Exactly 4 bits are inverted due to transmission errors. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5 | - Explain the process of IP address assignment using BOOTP and DHCP. | 5M | CO-3 | BL-2 |
|  | - Outline the primary functions of delivery, forwarding, and unicast routing protocols | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6 | Describe the process of address mapping using ARP and RARP, including the role of ARP cache and RARP servers. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Explain the difference between connectionless and connection-oriented protocols. | 5M | CO-4 | BL-2 |
|  | - Give the format of the UDP segment and mention when UDP is preferred over TCP. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 8 | How would you configure SCTP to optimize congestion control for a specific network scenario? Compare and contrast SCTP congestion control with TCP congestion control. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - Discuss in detail about File Transfer Protocol process. | 5M | CO-5 | BL-2 |
|  | - Illustrate architecture of E-Mail and discuss on its components. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |  |
| 10 | Compare and contrast the advantages and disadvantages of using a network-based firewall versus a host-based firewall. | 10M | CO-5 | BL-4 |

****