| Subject Code 22PC1AE303 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

DESIGN FOR AUTOMOTIVE COMPONENTS

(AE)

Time: 3 hours                                                                                 	             Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Analyze the significance of factor safety in design | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List three methods to reduce stress concentration in machine parts. | 2M | CO-2 | BL-1 |
|  | Differentiate between a butt weld and a fillet weld. | 2M | CO-3 | BL-1 |
|  | Distinguish between open and cross belt drives. | 2M | CO-4 | BL-2 |
|  | What is a sunk key, and how is it different from a saddle key? | 2M | CO-4 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - What do you understand by preferred numbers? Find out the numbers of the different basic series from 1 to 10. | 5M | CO-1 | BL-1 |
|  | - Describe the mechanical properties of engineering materials, such as strength, toughness, hardness, and fatigue resistance, and their importance in material selection. | 5M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 2. | - What are the factors to be considered for the selection of materials for the design of machine elements? | 5M | CO-1 | BL-1 |
|  | - The load on a bolt consists of an axial pull of 10 kN together with a transverse shear force of 5 kN. Find the diameter of bolt required according to 1. Maximum principal stress theory; 2. Maximum shear stress theory; 3. Maximum distortion energy theory. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Explain the following terms in connection with design of machine members subjected to variable loads:(a) Endurance limit, (b) Size factor, (c) Surface finish factor, and (d) Notch sensitivity. | 5M | CO-2 | BL-1 |
|  | b) A circular bar of 500 mm length is supported freely at its two ends. It is acted upon by a central concentrated cyclic load having a minimum value of 20 kN and a maximum value of 50 kN. Determine the diameter of bar by taking a factor of safety of 1.5, size effect of 0.85, surface finish factor of 0.9. The material properties of bar are given by: Ultimate strength of 650 MPa, yield strength of 500 MPa and endurance strength of 350 MPa. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Explain with a neat sketch Soderberg, Goodman and Gerber theories for fatigue strength. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | - Discuss on bolts of uniform strength giving examples of practical applications of such bolts | 5M | CO-3 | BL-2 |
|  | - A wall bracket, as shown in Fig. is fixed to a wall by means of four bolts. Find the size of the bolts and the width of bracket. The safe stress in tension for the bolt and bracket may be assumed as 70 MPa. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | A bracket, as shown in Fig., carries a load of 10 kN. Find the size of the weld if the allowable shear stress is not to exceed 80 MPa. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | A safety valve of 60 mm diameter is to blow off at a pressure of 1.2 N/mm2. It is held on its seat by a close coiled helical spring. The maximum lift of the valve is 10 mm. Design a suitable compression spring of spring index 5 and providing an initial compression of 35 mm. The maximum shear stress in the material of the wire is limited to 500 MPa. The modulus of rigidity for the spring material is 80 kN/mm2. Calculate: 1. Diameter of the spring wire, 2. Mean coil diameter, 3. Number of active turns, and 4. Pitch of the coil | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | a) What are the advantages and disadvantages of V-belt drive over flat belt drive? | 5M | CO-4 | BL-1 |
|  | b) A V-belt drive system transmits 100 kW at 475 r.p.m. The belt has a mass of 0.6 kg/m. The maximum permissible tension in the belt is 900 N. The groove angle is 38° and the angle of contact is 160°. Find minimum number of belts and pulley diameter. The coefficient of friction between belt and pulley is 0.2 | 5M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | a) How are the keys classified? Draw neat sketches of different types of keys and state their applications. | 5M | CO-4 | BL-2 |
|  | b) A 45 mm diameter shaft is made of steel with a yield strength of 400 MPa.A parallel key of size 14 mm wide and 9 mm thick made of steel with a yield strength of 340 MPa is to be used. Find the required length of key, if the shaft is loaded to transmit the maximum permissible torque. Use maximum shear stress theory and assume a factor of safety of 2. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10. | Design a bushed-pin type of flexible coupling to connect a pump shaft to a motor shaft transmitting 32 kW at 960 r.p.m. The overall torque is 20 percent more than mean torque. The material properties are as follows: a) The allowable shear and crushing stress for shaft and key material is 40 MPa and 80 MPa respectively. b) The allowable shear stress for cast iron is 15 MPa. c)The allowable bearing pressure for rubber bush is 0.8 N/mm2. d)The material of the pin is same as that of shaft and key. Draw a neat sketch of the coupling. | 10M | CO-4 | BL-3 |

***