| Subject Code 22PC1CE301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

DESIGN OF REINFORCED CONCRETE STRUCTURES

(CE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Note 	: IS 456 Code book, SP 16 Design charts are permitted to use

:  Assume any missing data suitably.

PART-A

1X24 = 24 M

| 1. | Design a Two-way slab Design a reinforced concrete slab for a room measuring 5m x 6m size. The slab is simply supported on all the four edges, with corners held down and carries a superimposed load of 3000 N/m2, inclusive of floor finishes. Use M20 mix, Fe 415 steel. Sketch the reinforcement details. | CO-3 | BL-4 |
| --- | --- | --- | --- |
| 2. | Design an isolated footing of uniform thickness of a reinforced concrete column bearing a vertical load of 400 kN and having a base of size 500 x 500 mm. The safe bearing capacity of soil may be taken as 120 kN/m2. Use M20 is concrete and Fe 415 steel. Sketch the reinforcement details. | CO-4 | BL-4 |

PART-B

3X12 = 36 M

| 3. | a) Write a short note on basic assumptions in limit state design and represent the stress-block parameters. b) A singly reinforced rectangular beam of size 230 mm x 500 mm effective depth is subjected to a factored moment of 175 kN/m. Find the reinforcement for flexure. Use M 20 concrete and Fe 415 grade steel. | CO-1 | BL-4 |
| --- | --- | --- | --- |
| 4. | A doubly reinforced concrete beam is 300 mm wide and 600 mm deep to the centre of tensile reinforcement. The compression reinforcement consists of 4 bars of 16 mm diameter, and is placed with its centre at a depth of 40 mm from the top. The tensile reinforcement consists of 4 bars of 20 mm diameter. The section is subjected to a bending moment of 100 kN-m. (Assume Steel grade Fe415, Concrete M20). | CO-1 | BL-3 |
| 5. | Determine the Moment of Resistance of T –beam as given in data . Width of Flange = 1200mm, width of Web = 300mm, Depth of flange = 100mm, Effective depth of the section = 560mm, Ast = 6 - # 28mm Use M20 & Fe415 grades. | CO-1 | BL-3 |
| 6. | Design the shear reinforcement for a beam section of width 230 mm and effective depth 500 mm. The factored shear force is 100 kN and it is reinforced with 3 Nos 16 mm diameter bars on the tension side at the critical section. Use M20 concrete and Fe 415 steel. | CO-1 | BL-4 |
| 7. | An R C rectangular column of size 230 mm × 300 mm is reinforced with 4 bars of 20 mm ɸ provided one at each corner with an effective cover of 60 mm. Examine the safety of the column. If it is subjected to Pu = 300 kN. Mux = 30 kNm. Assume M20 concrete and Fe415 grade steel.(Uniaxial Loaded) | CO-2 | BL-4 |

***