| Subject Code 22PC1EC302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

DIGITAL SIGNAL PROCESSING

(Common to EIE and ECE)

Time: 3 hours                                                                 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Find the z transform of the signal x[n] = (n-3) u[n-3]. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Determine the 2-point DFT of the given sequence x[n] = [2, 5]. | 2M | CO-2 | BL-2 |
|  | In Bilinear transformation if w=0.75 π maps to Ω=4.82 Then w=0.5 π maps to which value of Ω? | 2M | CO-3 | BL-2 |
|  | What is the sufficient condition for linear phase characteristics of FIR filter. | 2M | CO-4 | BL-2 |
|  | Define the expression for down sampled signal for the input x[n]=δ[n] .The down sample factor is L=2 | 2M | CO-5 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Represent Linear Constant Coefficient Difference (LCCD) equation y(n) = y(n - 1) - y(n - 2) + 2x(n) + x(n-1) using block diagram | 5M | CO-1 | BL-2 |
|  | b) Determine the discrete Fourier series representation for the given sequence - x[n] = cosπ4 n. - x[n] = cosπ4 n+sinπ4 n. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | A causal discrete –time LTI system is described by y[n] - (3/4)* y[n-1] + (1/8)* y[n-2] = x[n] Where x[n] and y(n) are the input and output of the system respectively. - Determine the system function H(z) - Find its impulse response h(n) | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | a) Deduce with mathematical expression to explain circular time shift and complex conjugate property of DFT. | 5M | CO-2 | BL-2 |
|  | b) For the given sequence x1(n) = (1, 2, 3, 1) and x2 (n) = (4, 3, 2, 2) using the four point DFT method determine the sequence x3(n) corresponding to the circular convolution of the sequences x1(n) and x2(n). | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 4. | Compute the eight –point DFT sequence X(n)=1 0≤n≤70 else using Radix-2 DIFFFT algorithm. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5. | a) What is frequency warping effect while design of IIR digital filter? | 5M | CO-3 | BL-1 |
|  | b) For the given difference equation y[n] = x[n] -2x[n-1] +y[ n-1] +0.5 y[n-2]. Realize the system using direct form-1 and direct form-2 structures. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | An IIR digital low pass filter is required to meet the following specifications: Passband ripple ≤0.5dB Passband edge frequency: 1.2 KHz Stopband attenuation: ≥40dB Stopband edge frequency: 2KHz Sample rate: 8KHz The filter is to be designed by performing bilinear transformation on an analog system function. Determine the order of Butterworth and Chebyshev filter that must be used to meet the specification in the digital implementation. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) Write a short note on window’s given below i. Hamming window ii. Rectangular window | 5M | CO-4 | BL-1 |
|  | b) Draw the Cascade form of structure for an FIR filter given by following equation H(z) = [1+0.5*z-1 +z-2 ] [1+0.25 z-1 ] | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8. | A low pass filter is designed to obtain the following frequency response: Hdejw= e-j3w -3π 4 ≤w≤ 3π40 otherwise Calculate the filter coefficient hd(n) and determine the frequency response of the FIR filter if hamming window is used. | 10M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9. | - Explain the sampling rate conversion by rational factor I/D. | 5M | CO-5 | BL-2 |
|  | - What is the cause of overflow oscillations? Mention two methods to prevent overflow. | 5M | CO-5 | BL-1 |
| OR |  |  |  |  |  |
| 10. | An LTI system characterized difference equation y(n)=0.68 y(n-1)+0.15x(n). The input signal has a range of -5v to +5v represented by 8bits. Find the variance of quantization noise at the output. | 10M | CO-5 | BL-4 |

***