| Subject Code 22PC1CE303 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

ESTIMATION AND COSTING

(CE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | List out atleast 2 deductions for openings in masonry wall. | 2M | CO1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Lead and lift | 2M | CO2 | BL-1 |
|  | State the importance of rate analysis. | 2M | CO1 | BL-1 |
|  | What is earnest money? | 2M | CO3 | BL-1 |
|  | Differentiate between Price and Cost | 2M | CO4 | BL-1 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Explain different types of estimates. Mention their purpose. | 5M | CO1 | BL-2 |
|  | b) Prepare a preliminary estimate of a four storeyed office building having a carpet area of 3500m2 for obtaining the administrative approval of the Government using the following data. It may be assumed that 25 percent of built up area will be taken up by the corridors, verandahs, lavatories, stair cases etc. and 10% of the built-up area will be occupied by walls. i) Plinth area rate is – ₹ 2570 / m2. ii) Extra due to deep foundation at site – 1% of building cost. iii) Extra for special architectural treatment – 2% of building cost. iv) Extra for water supply and sanitation – 8% of building cost. v) Extra for electrical installations – 15% of building cost. vi) Other services – 6.5% of building cost. vii) Contingencies – 3.5% of overall cost. viii) Super vision charges – 10% overall cost. | 5M | CO1 | BL-3 |
| OR |  |  |  |  |
| 2. | Prepare a detailed estimate of the following RCC works of a RCC framed building from the given section, elevation and plan (Fig. 1-A, B, C) i) Plinth beams ii) Columns iii) Roof slab. | 10M | CO1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | Estimate the quantity of earth work for portion of road work from the following data using Mid-sectional area method. Formation width = 4m, side slope 2:1 filling, side slope 1.5:1 in cutting. The length of each chainage is 40m. | 10M | CO2 | BL-3 |
| OR |  |  |  |  |
| 4 | Explain in detail about the three cases of canal structures with neat sketches. | 10M | CO2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Prepare rate analysis for 1 cu.m of Brick Masonry in C.M (1:6) in super structure with 20x10x10 cm. Labour gang : Head mason – ¼ Nos, Mason-2 Nos, Mazdoor-10 Nos, Bhisti–2 Nos for 10 m3 of concrete. | 5M | CO1 | BL-3 |
|  | - Prepare an analysis of rates for cement concrete 1:2:4 in the foundation of the floor with brick ballast 40mm (1½ ”) thick Labour gang : Head mason-1/2 Nos, Mason-8 Nos, Mazdoor-15 Nos, Bhisti-2 Nos for 10m3 of Brick work. | 5M | CO1 | BL-3 |
| OR |  |  |  |  |
| 6 | Estimate the quantity of steel used in the construction of R.C.C. Roof slab of 4 m clear span and 5 m long from the given drawings in the below figure. Assume suitable missing data if necessary. | 10M | CO1 | BL-5 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain how partnerships or joint ventures are managed in contracts involving more than two parties, and Discuss the challenges | 4M | CO-4 | BL-2 |
|  | - Explain the components of notice inviting tender with example. | 6M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | - Define bidding. Discuss the different types of bidding methods. | 4M | CO3 | BL-4 |
|  | - Differentiate percentage rate contract and cost plus percentage rate contract. | 6M | CO3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - Calculate the monthly rent for a property with an annual gross income of ₹2,40,000, a maintenance cost of ₹40,000, and a property tax of ₹20,000. | 5M | CO4 | BL-3 |
|  | - Calculate the value of a property with a net annual income of ₹50,000 for a long period and a capitalization rate of 8%. | 5M | CO4 | BL-3 |
| OR |  |  |  |  |
| 10 | Discuss the rental method of property valuation. Provide a step-by-step example calculation. | 10M | CO4 | BL-4 |

Fig. 1-A

Fig. 1-B

Fig. 1-C