| Subject Code 22OE1CS301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, Dec. 2024/Jan.2025

FUNDAMENTALS OF COMPUTER NETWORKS

(Open Elective)

Time: 3 hours                                                                                 		    Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A				           5X2=10M

|  | Define Multiplexing. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the difference between flow control and error control? | 2M | CO-2 | BL-1 |
|  | Define the role of ARP in address mapping. | 2M | CO-2 | BL-2 |
|  | How quality of service is achieved. | 2M | CO-3 | BL-2 |
|  | List any two differences between HTTP and HTTPS. | 2M | CO-4 | BL-1 |

PART-B				         5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - What is a virtual circuit network, and how does it differ from a datagram network? | 5M | CO-1 | BL-1 |
|  | - Explain how addressing works in networking and the importance of IP addressing schemes. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Explain the responsibilities of each layer in the ISO/OSI reference model. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Explain various channelization methods in the data link layer. | 5M | CO-2 | BL-2 |
|  | - A sender wants to send the binary data 1101011011 using a CRC with the generator polynomial G(x)=x3+x+1. i) What is the CRC code (remainder) after appending three zeros to the data? ii) What is the final transmitted data? | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | - Explain the flow control mechanism using the Sliding window protocol. | 5M | CO-2 | BL-2 |
|  | b) Discuss the specifications of Gigabit Ethernet. | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | - Draw the format of the IPv4 header? Describe the significance of each field. | 5M | CO-2 | BL-2 |
|  | - Explain the significance of the Border Gateway Protocol (BGP). | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | - Analyze Disjkstra’s algorithm with an example. | 5M | CO-2 | BL-4 |
|  | b) What is the significance of Bridges? Explain the different types of Bridges. | 5M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Discuss the following for Transmission Control Protocol - TCP connection management modeling - TCP transmission policy - TCP segment header. | 5M | CO-3 | BL-2 |
|  | - Explain the architecture, services, and message format of Electronic Mail. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | - Discuss congestion control mechanisms. | 5M | CO-3 | BL-2 |
|  | - Explain the functionality of DNS. | 5M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Write short notes on the following. i) HTTP ii) WWW | 5M | CO-4 | BL-2 |
|  | - Discuss the role of IPSec in ensuring secure communication over IP networks. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | - Discuss various types of security attacks. | 5M | CO-4 | BL-2 |
|  | - Compare SSL and VPN in terms of their application and security mechanisms. | 5M | CO-4 | BL-3 |

***