| Subject Code 22PE1EE320 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

FUNDAMENTALS OF SIGNALS AND SYSTEMS

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A				              5X2=10M

|  | In the case of signals and vectors what does orthogonality imply? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Does Fourier transform exist for all aperiodic functions? If not, what are the conditions to be satisfied for the existence of Fourier transform? | 2M | CO-2 | BL 2 |
|  | Mention the relation between Fourier transform and Laplace transform. | 2M | CO-2 | BL 2 |
|  | What is system bandwidth, what does it indicate? | 2M | CO-3 | BL 2 |
|  | What are the advantages and limitations of Z-transform? | 2M | CO-2 | BL 2 |

PART-B				              5X10=50M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) Find the even and odd components of the following signals - x(t) = 1+2t+3t2+4t3 (ii) x(n) = {5,4,3,2,1} | 5M | CO-1 | BL-2 |
|  | b) Define orthogonal signal space and bring out clearly its application in representing a signal? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | a) Find whether the given signal is periodic or not, if periodic find its fundamental time period xt=1+4cos20πt+5sin25πt | 5M | CO-1 | BL-2 |
|  | b) Calculate the energy associated with the signal xt=e-3t U(t). | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | Determine the Fourier transform of the periodic pulse function shown in fig. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | Determine the trigonometric form of Fourier series of the signal shown in fig. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | a) Determine the Laplace transform of the following signal using properties of Laplace transform | 5M | CO-2 | BL-3 |
|  | b) Check whether the following system is static or dynamic, linear or non linear, causal or non causal and time invariant or time variant. y(n) = 2x(n+2) – x(n-2) | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6. | a) Find the inverse Laplace transform of X(s) = 4s+2(s+4) if the ROC is -2 > Re(s) > -4. | 5M | CO-2 | BL-3 |
|  | b) Find convolution of the given signals xt=e-atUt, ht=Ut using graphical approach. | 5M | CO-4 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) Obtain the conditions for distortion less transmission through a system? | 5M | CO-3 | BL-2 |
|  | b) Perform cross correlation of the sequences x(n) = {1,1,2,2) and y(n) = {1,3,1} | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8. | a) Explain causality and physical realizability of a system and hence give Paley-Wiener criterion. | 5M | CO-3 | BL-3 |
|  | b) Mention the properties of correlation function. | 5M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | a) Define Nyquist sampling criterion and find the Nyquist sampling rate for xt=Cos150πt+Sin200πt. | 5M | CO-5 | BL-3 |
|  | b) Find the Z transform and ROC of x(n) = 2(56)n u(-n-1) + 3(12)2n u(n) sketch the ROC and pole zero location. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 10. | a) With the help of graphical example explain sampling theorem for band limited signals? | 5M | CO-5 | BL-3 |
|  | b) Fine inverse Z-transform to get all possible signals x(n) associated with Z-transform given below. X(z) = (14)z-11-12z-1[1-14z-1] | 5M | CO-2 | BL-3 |

***