| Subject Code 22PC1EI301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

INDUSTRIAL PROCESS CONTROL INSTRUMENTATION

(EIE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | State practical applications for a Venturi meter. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Describe steady-state error in a control system. | 2M | CO-2 | BL-1 |
|  | What is an Internal Model Control (IMC). | 2M | CO-2 | BL-2 |
|  | State the purpose of adaptive control. | 2M | CO-3 | BL-2 |
|  | Define the term Control valve sizing. | 2M | CO-4 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Describe the working of Differential Pressure Transmitter (DPT). | 5M | CO-1 | BL-2 |
|  | - Explain the construction and working of a turbine flow meter. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | With suitable diagrams, explain the principles of Venturi and orifice meters. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Discuss the importance of mathematical modeling in process control. | 5M | CO-2 | BL-2 |
|  | - Illustrate interacting and non-interacting processes with examples. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | Compare open-loop and closed-loop control for a level process in terms of stability and performance. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | - Explain the working principle of a proportional controller and its effect on system response. | 5M | CO-2 | BL-2 |
|  | - Discuss the advantages and limitations of using PID controllers in industrial applications. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | Explain PID controller tuning for FOPDT process using the PRC procedure. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Describe the working principle of cascade control with an example. | 5M | CO-3 | BL-2 |
|  | - Explain the concept of inferential control with a suitable diagram. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | Implement a feed-forward control system with feedback compensation for an industrial application. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Distinguish between a globe valve and a butterfly valve. | 5M | CO-4 | BL-4 |
|  | - Illustrate the working principle of hydraulic actuators in control systems. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Explain the role of a control valve positioner in a feedback control loop and describe how it enhances the performance of the valve. | 10M | CO-4 | BL-2 |

***