| Subject Code 22PE1EC301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

INFORMATION THEORY AND CODING

(ECE)

Time: 3 hours                                                                                       Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define Kraft’s Inequality? | 2M | CO-1 | BL1 |
| --- | --- | --- | --- | --- |
|  | Construct the binary field under modulo-2 addition and modulo-2 multiplication. | 2M | CO-3 | BL2 |
|  | Consider a code C with two code words = {0010, 0011}. Find the Hamming weight for each code and value of Hamming distance between 2 code words. | 2M | CO-2 | BL2 |
|  | Define cyclic codes? | 2M | CO-4 | BL1 |
|  | Define constraint length and free distance of a convolutional code? | 2M | CO-5 | BL1 |

PART-B				5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Determine the Huffman coding and Shannon-Fano coding for the following messages with their probabilities given p(x1)=0.05, p(x2)=0.15, p(x3)=0.2, p(x4)=0.05, p(x5)=0.15, p(x6)=0.3, p(x7)=0.1. Find the efficiency and redundancy of the code. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 2. | State Shannon's Source Coding Theorem and explain its implications for data compression. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Explain Vector spaces of a Field? | 5M | CO-3 | BL-2 |
|  | - Explain Matrices over GF(2𝑚)? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 4. | Explain Construction of Galois Fields GF(2𝑚) with an example | 10M | CO-3 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | Explain Error Detecting and error correcting capabilities with examples? | 10M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 6. | The parity matrix for a (7,4) linear block code is given below. P=110011111101 i) Find generator and parity check matrices. ii) Find the code words for message vector 1001, 1010, 0001, 1110. iii) Draw the encoder circuit. iv) Illustrate the decoding of the received vector corresponding to the message vector 1001, if it is received with 5th bit in error. | 10M | CO-4 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - How is Error detection different from syndrome Computation? | 5M | CO-4 | BL-2 |
|  | - Explain Parity Check matrices in details with an example? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | Find a generator polynomial for a (7,4) cyclic code and find the code vectors for the following data vectors 1010, 0011, 1000 and 1100 both in systematic and non-systematic form. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | For (2, 1, 2) convolutional encoder with g(1)=(101), g(2)=(111). i) Draw the encoder diagram. ii) Write state transition table. iii) Draw state diagram. iv) Draw the code tree v) Trace output for the input sequence 10011 from the code tree. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10. | Describe the Structural properties of Convolutional codes. | 10M | CO-4 | BL-2 |

***