| Subject Code 22PC1ME303 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

MECHANICAL ENGINEERING DESIGN

(ME)

Time: 3 hours                                             			      	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

Design Data Book is permitted.

PART-A

5X2=10M

|  | Which theory of failure is applicable to brittle material? Why? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Differentiate Endurance limit and Fatigue Limit. | 2M | CO-2 | BL-2 |
|  | What is meant by bolt of uniform strength? | 2M | CO-2 | BL-1 |
|  | Define and distinguish the terms module and circular pitch in Gears | 2M | CO-4 | BL-1 |
|  | Why ball and roller bearings are called ‘antifriction’ bearings? | 2M | CO-3 | BL-1 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - What are standards? Explain its significance in machine design? | 5M | CO-1 | BL-1 |
|  | - A shaft is designed based on maximum distortion energy theory with a factor of safety of 2.0. The material used is 30C8 steel with a yield stress of 310 MPa. It is subjected to an axial load of 40 KN. Determine the maximum torque capacity. Diameter of shaft is 20 mm | 5M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 2 | The shaft of an overhang crank subjected to a force P of 1 kN is shown in figure. The shaft is made of plain carbon steel 45C8 and the tensile yield strength is 380 N/mm2. The factor of safety is 2. Determine the diameter of the shaft using the maximum shear stress theory. | 10M | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |
| 3 | - A steel connecting rod made of AISI 8650 steel, is to be subjected to a reversed axial load of 180kN. Determine the required diameter of the rod, using a factor of safety N = 2. Take properties are Sut = 1.07 GN/m2, Syt = 910 MN/m2. Assume that the endurance limit for reversed bending is half the ultimate tensile strength. | 5M | CO-2 | BL-1 |
|  | - Explain the Low cycle and High cycle fatigue? Also define Notch Sensitivity factor. | 5M | CO-2 | BL-1 |
| OR |  |  |  |  |
| 4 | Explain Goodman’s and Soderberg’s criteria for design of materials under fluctuating loads. | 10M | CO-2 | BL-1 |
| UNIT-III |  |  |  |  |
| 5 | Figure shows the bracket used in a jib crane to connect the tie rod. The maximum force in the tie rod is 5 kN, which is inclined at an angle of 30° with the horizontal. The bracket is fastened by means of four identical bolts, two at A and two at B. The bolts are made of plain carbon steel 30C8 (Syt = 400 N/mm2) and the factor of safety is 5. Assume maximum shear stress theory and determine the size of the bolts. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6 | A double – riveted double – strap butt joint is used to connect two plates, each of 12 mm thickness, by means of 16 mm diameter rivets having a pitch of 48 mm. The rivets and plates are made of steel. The permissible stresses in tension, shear, and compression are 80, 60, and 120 N/mm2 respectively. Find the efficiency of the joint. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | A helical cast steel gear with 300 helix angle has to transmit 35 KW at 1500 rpm. If the gear has 24 teeth, determine the necessary module, pitch diameter and face width for 200 full depth teeth. The static stress for cast steel may be taken as 56 MPa. The width of face may be taken as 3 times the normal pitch. What would be the end thrust on the gear? | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | The layout of a leather belt drive transmitting 15 kW of power is shown in Fig. The centre distance between the pulleys is twice the diameter of the bigger pulley. The belt should operate at a velocity of 20 m/s approximately and the stresses in the belt should not exceed 2.25 N/mm2. The density of leather is 0.95 g/cc and the coefficient of friction is 0.35. The thickness of the belt is 5 mm. Calculate: (i) the diameter of pulleys; (ii) the length and width of the belt; and (iii) the belt tensions. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | A full journal bearing having 90 mm dia and 150 mm long has a radial load resulting in pressure of 2 MPa per unit projected area. The speed of shaft is 500 rpm. The bearing operates in SAE20 oil at 50oC, calculate (i) The minimum film thickness, (ii) Heat loss due to friction and (iii) Check requirement for artificial cooling. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 10 | Design a CI piston for single acting 4-stroke engine for the following specifications: Cylinder bore = 100mm Stroke = 120mm Maximum gas pressure = 5 MPa Break mean effective pressure = 0.65 MPa Mechanical efficiency = 80% HCV of fuel = 42× 103 KJ/Kg Fuel consumption = 0.23 Kg/KW/hr Speed = 2200 rpm. Any other data required for the design may be assumed. | 10M | CO-3 | BL-4 |

****