| Subject Code 22PE1CE301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

PAVEMENT ANALYSIS AND DESIGN

(CE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Determine the radius of relative stiffness for slab thickness 20cm, E=2.1X 105 Kg/cm2, k=7.5kg/cm3, µ = 0.13 | 2M | CO-3 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the desirable properties of soil for pavement construction | 2M | CO-1 | BL-1 |
|  | Calculate the equivalent radius of resisting section of 20 cm thick slab , given that the radius of contact area is 15 cm. | 2M | CO-3 | BL-1 |
|  | Draw a typical section of pavement as per three layer theory | 2M | CO-4 | BL-1 |
|  | What do you mean white topping | 2M | CO-2 | BL-1 |

PART-B					5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Discuss the factors affecting design of pavements. | 5M | CO-1 | BL-2 |
|  | - Explain how axle load survey is performed for the determination of VDF. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | The following data were obtained from an axle survey. Determine VDF based on a standard axle load of 80KN. Axle load(kN); 30-40; 40-50; 50-60; 60-70; 70-80; 80-90. No. of repetitions (%); 10000; 8000; 4000; 2500; 1850; 1000. Axle load(kN); 90-100; 100-110; 110-120; 120-130; 130-140; 140-150. No. of repetitions (%); 820; 570; 350; 280; 190; 100. | 10M | CO-3 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | a) Discuss three layer elastic theory for design of flexible pavement. | 5M | CO-2 | BL-1 |
|  | b) Explain briefly the principle of Burmister two layer theory and mention the advantages over the single layer theory for the analysis of flexible pavements. | 5M | CO-2 | BL-1 |
| OR |  |  |  |  |
| 4. | Determine the wheel load and warping stress from wheel load and warping stress equations at edge and corner region of the slab. The design data are given below. Design wheel load, P=7000kg, k=7.5kg/cm3 Lx=11.5m Ly=4.0m Elastic modulus of pavement material, E=3x105 kg/cm2 Thermal coefficient of CC per deg. C, =1x10-5 Temperature difference t = 150C ; a = 15cm; µ = 0.15 | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Enumerate the various approaches of flexible pavement design. Briefly indicate the basis of design in each case | 5M | CO-3 | BL-1 |
|  | - Write an overview of AASHTO method of flexible pavement design | 5M | CO-3 | BL-1 |
| OR |  |  |  |  |
| 6. | Estimate the effective CBR of subgrade modulus of the flexible pavement as per IRC 37-2018 using following data: (i) CBR of the soil used in the upper 500mm of embankment is 8% and borrow soil used for preparing the 500mm thick compacted sub-grade above embankment is 20% (ii) Given surface deflection as per IITPAVE is 1.41mm (iii) µ = 0.35, a=150.8mm (iv) Load is 40,000N; pressure is 0.5MPa | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Enumerate the functions and different types of joints in concrete pavements | 5M | CO-3 | BL-2 |
|  | b) A cement concrete pavement has a thickness of 26cm and lane width of 3.5m.Design the tie bars along the longitudinal joints using the data given below. Allowable working stress in steel tie bars, SS=1250kg/cm2 Unit weight of CC,W=2400kg/m3 Maximum value of coefficient of friction=1.2 Allowable tensile stress in deformed tie bar, SS=2000kg/cm2 Allowable bond stress in deformed bars, Sb=24.6kg/cm2 | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | Discuss the design steps of Rigid pavements as per IRC: 58-2015? | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Describe the steps involved in designing a rigid overlay over an existing rigid pavement | 5M | CO-4 | BL-2 |
|  | - Benkelman beam deflection studies were carried out on 15points of flexible pavement suing standard truck with single axle dual wheel. The deflection values obtained after necessary leg corrections are as follows: 1.40, 1.32, 1.25, 1.35, 1.35, 1.48, 1.60, 1.65, 1.55, 1.45, 1.40, 1.36, 1.46, 1.50, 1.52 and 1.45mm. If the pavement temperature is 300C and moisture correction is 1.3. Design traffic is 1768 commercial veh. /day. Allowable deflection (Da = 1.0mm) determine overlay thickness of granular material. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | With neat sketches, explain the working principle of the Benkelman beam used to measure rebound deflection of flexible pavements | 10M | CO-4 | BL-1 |

***