| Subject Code 22PC1EE301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

POWER ELECTRONICS

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | List the advantages of high band gap materials. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define firing angle. Mention the range of firing angle for a rectifier. | 2M | CO-1 | BL-1 |
|  | Define duty ratio. What is the effect of change in duty ratio in dc-dc converters? | 2M | CO-3 | BL-2 |
|  | Write the significance of frequency modulation and modulation index for inverters. | 2M | CO-3 | BL-2 |
|  | Mention the industrial applications of cycloconverters. | 2M | CO-2 | BL-2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - With relevant circuit and graphs, explain the construction and static characteristics of power BJT. | 5M | CO-1 | BL-2 |
|  | - Give the comparison between BJT, MOSFET and IGBT. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | With necessary circuit and waveforms, analyse the characteristics of SCR during turn-on and turn-off condition. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | - With necessary circuit and waveforms, explain the principle of operation of single phase full controlled bridge rectifier feeding RL load and derive the expression for the average output dc voltage. | 5M | CO-2 | BL-2 |
|  | - Explain the effect of source inductance on the performance of the ac to dc converter. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | With necessary circuit and waveforms, explain the working of three phase full controlled bridge rectifier feeding RL load. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | - Derive the relation for average of output voltage of Boost converter with necessary circuits and waveforms. | 5M | CO-2 | BL-3 |
|  | - Explain the duty ratio control method to control the output voltage of DC-DC converters. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | The buck dc-dc converter has the following parameters: Vs = 50 V, D = 0.4, L = 400 µH, C = 100 µF, f = 20 kHz, R = 20Ω. Assuming ideal components, calculate (a) the average output voltage Vo, (b) the maximum and minimum inductor current, and (c) the output voltage ripple. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Explain the principle of operation of the single-phase full bridge inverter with RL load along with suitable waveforms. Also write the expression for load current as function of time for first 2-half cycles of output voltage. | 5M | CO-2 | BL-2 |
|  | - Explain the following, i) Single pulse width modulation, ii) Multiple pulse width modulation. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | With the help of neat circuit diagram and waveforms, explain briefly the operation of a three-phase bridge inverter with resistive load in 1200 conduction mode. | 10M | CO-2 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Describe the basic principle of working of a step-up cycloconverter with resistive load in mid-point type configuration. | 5M | CO-2 | BL-2 |
|  | - Derive the relation for the rms output voltage of single phase ac voltage controller with RL load. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 10. | For a single-phase ac voltage regulator feeding a resistive load, draw the waveforms of source voltage, gating signals, output voltage, output current and voltage across SCRs. Describe its working with reference to the waveforms drawn. | 10M | CO-2 | BL-2 |

***