| Subject Code 22PC1EE302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

POWER SYSTEMS-II

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | What is an Incidence Matrix, and what does it represent in power system analysis? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the role of the acceleration factor in the Gauss-Seidel method? | 2M | CO-2 | BL-2 |
|  | Define the term 'Fault MVA' in the context of symmetrical fault analysis. | 2M | CO-3 | BL-1 |
|  | Give the classification of various types of Faults in a power system network. | 2M | CO-3 | BL-2 |
|  | Write the expression for swing equation used in power system stability . | 2M | CO-4 | BL-1 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - The parameters of a 4 – bus system are as follows. Bus code; Line impedance (pu); Half Line Charging admittance (pu). 1- 2; 0.2 + j 0.8; j 0.02. 2- 3; 0.3 + j 0.9; j 0.03. 2- 4; 0.25 + j 1.0; j 0.04. 3- 4; 0.2 + j 0.8; j 0.02. 1- 3; 0.1 + j 0.4; j 0.01. Draw the network and find the bus admittance matrix using direct inspection method. | 5M | CO-1 | BL-2 |
|  | - Develop step-by-step algorithm for Zbus building considering all possible modifications for a network containing no mutual and no phase shifting transformers | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | Develop Ybus matrix by direct inspection method for the given power system shown in figure. All the line self impedances are given in per unit. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | - Writ the advantages and disadvantages of N -R method over Gauss Seidel method. | 5M | CO-2 | BL-3 |
|  | - Define bus in power system? Explain the types of buses and their significance. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | Derive the static load flow equations for an n-bus system | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | - Why do we use reactors in the power system? Discuss their advantages. | 5M | CO-3 | BL-1 |
|  | - Explain in detail the importance of per unit(P.U) system used in power systems. Write the expression for per unit impedance. | 5M | CO-3 | BL-1 |
| OR |  |  |  |  |  |
| 6 | Prove that the transformer per unit reactance is same on primary and secondary sides. Write the expression for new per unit impedance in terms of old and new bases. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Derive the equation of sequence impedance of unloaded synchronous generator and sketch the sequence networks. | 5M | CO-4 | BL-1 |
|  | - The line currents in a 3-phase supply to an unbalanced load are Ia = - 10+ j 20, Ib = 12 - j10 and Ic = - 3 - j5 amperes respectively. The phase sequence is abc. Determine the sequence components of currents. | 5M | CO-4 | BL-1 |
| OR |  |  |  |  |  |
| 8 | Calculate the sub-transient fault current in each phase for a dead short circuit when a single phase to ground fault occurs after the second transformer for the system shown in figure. All the reactances are given in p.u. on the generator base. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - How the equal area criterion is applied when there is a sudden (i) Sudden increase in mechanical power input, and (b) Three-phase fault at one end of line in a double circuit line. | 5M | CO-4 | BL-1 |
|  | - A generator supplies 1.0 pu. power to an infinite bus as shown in figure. The terminal voltage and infinite bus voltage are 1.0 pu. All the reactances are on a common base. Determine steady state stability limit when both lines are in and when one line is switched off. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10 | Write short notes on - transient stability improvement methods - swing equation | 10M | CO-4 | BL-1 |

***