| Subject Code 22PC1ME301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

THERMAL ENGINEERING

(ME)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Compare between two-stroke and four-stroke engines. | 2M | CO-1,2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the basic need for the ignition system? | 2M | CO-2,3 | BL-2 |
|  | List out the IC engine performance measurement parameters? | 2M | CO-2,3 | BL-2 |
|  | Define volumetric efficiency of an air compressor? And write the expression for volumetric efficiency. | 2M | CO-3,4 | BL-2 |
|  | What is ton of refrigeration? | 2M | CO-3,4 | BL-1 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the brief classification of I.C. Engines. | 5M | CO-1,2 | BL-2 |
|  | - Explain the Valve timing diagram of 4 Stroke engine with neat sketch. | 5M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
| 2. | Illustrate the following with neat sketches: (i) time loss factor, (ii) heat loss factor, (iii) exhaust blow down factor and (iv) rubbing losses. | 10M | CO-1,2 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - With the help of a neat diagram, explain the working of fuel injection system. | 5M | CO-1,2 | BL-2 |
|  | - List out the functional requirements of a good lubrication system. | 5M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
| 4. | Explain the stages of combustion in SI engines with neat sketches? What are the various factors influencing the flame speed. | 10M | CO-2,3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Enumerate the factors affecting the delay period in C.I. engine. | 5M | CO-2,3 | BL-2 |
|  | - Discuss in detail the phenomenon of diesel knock and list the methods to avoid it. | 5M | CO-2,3 | BL-2 |
| OR |  |  |  |  |
| 6. | During the trail on a single cylinder the 4-S cycle oil engine fitted with a rope brake, the following readings are taken. Effective diameter of Brake wheel = 600 mm, Dead load on brake = 200 N; spring balance reading = 30 N; speed = 450 r.p.m; Area of indicator diagram is 400 mm2 ; length of indicator diagram = 60 mm; spring scale = 1.1 bar per mm. Bore = 100 mm; stroke = 150 mm; Quantity of oil = 0.815 kg/hr. Calorific value of oil = 42000 kJ/kg. Calculate the brake power, indicated power, mechanical efficiency, brake thermal efficiency and brake specific fuel consumption and indicated thermal efficiency. | 10M | CO-2,3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Elucidate the general layout of electric vehicle. | 5M | CO-1,3 | BL-2 |
|  | - List out the advantages and disadvantages of electric vehicle. | 5M | CO-1,3 | BL-2 |
| OR |  |  |  |  |
| 8. | A single acting reciprocating air compressor has cylinder diameter and stroke of 200 mm and 300 mm respectively. The compressor sucks air at 1 bar and 270 C and delivers at 8 bar while running at 100 r.p.m. Find Indicated power of the compressor and mass of air delivered by the compressor. Compression process follows the law of PV1.3 = constant and take R=0.287 kJ/kg K. | 10M | CO-3,4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Define the terms i) Refrigeration ii) Co-efficient of performance. | 5M | CO-3,4 | BL-2 |
|  | - State the advantages of vapour compression system over air refrigeration system. | 5M | CO-3,4 | BL-2 |
| OR |  |  |  |  |
| 10. | A simple vapour compression plant produces 5 tons of refrigeration. The enthalpy values at inlet to compressor, at exit from the compressor and at exit from the condenser are 183.19, 209.41 and 74.59 kJ/kg respectively. Estimate (i) The refrigerant flow rate (ii) The COP (iii) The power required to drive the compressor, (iv) The rate of heat rejection to the condenser. | 10M | CO-3,4 | BL-4 |

***