| Subject Code 22PE1CE304 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

TRAFFIC ENGINEERING

(CE)

Time: 3 hours                                                                                 	             Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Use of HCM Manual is allowed.

PART-A					5X2=10M

|  | List the objectives of speed studies | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define practical capacity | 2M | CO-2 | BL-1 |
|  | List different on-site parking methods. | 2M | CO-3 | BL-1 |
|  | Define Cycle time. | 2M | CO-3 | BL-1 |
|  | State the detrimental effects of traffic on environment. | 2M | CO-4 | BL-1 |

PART-B				       5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Following is the data observed for a given road stretch. Calculate PHF for 15-minute interval. Time; 4.00-4.15; 4.15-4.30; 4.30-4.45; 4.45-5.00; 5.00-5.15; 5.15-5.30; 5.30-5.45; 5.45-6.00. Volume (pcu); 90; 85; 80; 95; 120; 110; 145; 140. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | a) Explain the relation between speed, flow and density with neat sketches. | 5M | CO-1 | BL-2 |
|  | b) List the objectives of volume studies and discuss various methods | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | Explain the concept of ‘EEE’ related to accidents. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | Explain the various factors affecting Level of Service and Capacity of the road and with the help of a neat sketch, elaborate different types of Level of Service. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | Classify different types of parking facilities. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | The parking survey data collected from a parking lot by license plate method is as shown in the below table. Find the average occupancy, average turn-over, parking load, parking capacity and efficiency of the parking lot. Bay; Time. 0 – 15; 15 - 30; 30 – 45; 45 - 60. 1; 5035; 5035; -; 4032. 2; 2542; 6254; 6254; 3214. 3; 8745; 8745; 8745; 8745. 4; 9874; 5410; -; -. 5; 5879; -; 1245; 5487. 6; 2587; 2587; 4587; 4587. 7; 3574; 3574; 1458; 1458. 8; -; -; 4587; 6589. 9; 1458; 2587; -; 1452. 10; 2587; -; 3521; 1254. 11; 6589; 4587; 2563; 1452. 12; -; 2541; 5874; 6985. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - List out Warrants for signalization. | 5M | CO-3 | BL-2 |
|  | - Define offset. Explain different methods of signal coordination in detail. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | The normal flow of traffic on crossroads A and B are 400 and 250 vehicles per hour respectively. The saturation of flow for roads A and B are estimated as 1250 and 1000 vehicles per hour respectively. The all-red time for pedestrians to cross is 12 seconds. Design a two-phase traffic system by Webster's method. Use (L=nl+R) equation to find “L”. Assume lost time per phase as 2sec. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | Identify and explain the major air pollutants produced by highway traffic, along with the corresponding remedial measures. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Summarize the impact of noise, its sources, and the strategies for mitigating noise pollution. | 10M | CO-4 | BL-2 |

***