| Subject Code 22PC1AE302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

VEHICLE DYNAMICS

(AE)

Time: 3 hours                          	          			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define Lumped Mass | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Differentiate Power vs Traction Limited Acceleration | 2M | CO-2 | BL-2 |
|  | List the drag components acting on vehicle body | 2M | CO-3 | BL-1 |
|  | Define logarithmic decrement. | 2M | CO-4 | BL-1 |
|  | Define handling. | 2M | CO-5 | BL-1 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | - Illustrate the Vehicle-Fixed Coordinate System |  |  | 5M | CO-1 | BL-2 |
|  | - Enumerate the importance of Lumped Mass in Vehicle Dynamics Modelling |  |  | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |  |
| 2. | Derive the expression for dynamic axle loads for the following cases. i) Static Loads on level ground ii) Low-speed acceleration iii) Loads on grades. |  | 10M |  | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |  |  |
| 3. | - Derive the braking performance for the constant deceleration and deceleration with wind resistance. |  | 5M |  | CO-2 | BL-2 |
|  | - Explain Tyre-Road friction |  | 5M |  | CO-2 | BL-2 |
| OR |  |  |  |  |  |  |
| 4. | Find the traction-limited acceleration for the rear-drive passenger car with and without a locking differential on a surface of moderate friction level. The information that will be needed is as follows Weight; Front, 952 kg; Rear, 840 kg total 1792 kg. C.G. Height; 0.53 m; Wheelbase, 2.74 m. Coefficient of friction; 0.62; Tread, 1.5 m. Final drive ratio; 2.9; Tire size, 0.33m. Roll stiffness; Front, 1559 N-m /deg.; Rear, 380 N-m/deg.. | 10M |  | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |  |
| 5. | - Explain in detail mechanics of air flow | 5M |  |  | CO-3 | BL-2 |
|  | - Explain in detail pressure distribution on a vehicle | 5M |  |  | CO-3 | BL-2 |
| OR |  |  |  |  |  |  |
| 6. | a) Illustrate the aerodynamics aids. b) With neat diagram discuss the crosswind sensitivity. | 5M 5M |  |  | CO-3 CO-3 | BL-3 BL-3 |
| UNIT-IV |  |  |  |  |  |  |
| 7 | - Explain in detail the terminology of vibrations | 5M |  |  | CO-4 | BL-2 |
|  | - Illustrate excitations during ride on vehicle | 5M |  |  | CO-4 | BL-2 |
| OR |  |  |  |  |  |  |
| 8, | The mass of a spring-mass-dashpot system is given an initial velocity 5ωn, where ωn is the undamped natural frequency of the system. Find the equation of motion for the system, when (i) ζ = 2.0, (ii) ζ = 1.0, (i) ζ = 0.2. | 10M |  |  | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |  |  |
| 9, | Derive the expression for high speed cornering and discuss under steer gradient. | 10M |  |  | CO-5 | BL-4 |
| OR |  |  |  |  |  |  |
| 10, | A car has a weight of 862 kg on the front axle and 704 kg on the rear with a wheelbase of 2.6 m. The tires have the following cornering stiffness values as represented in table below. Both front and rear tyres are having same cornering stiffness and cornering coefficient. Determine the following cornering properties for the vehicle: a. Ackerman steer angles for 152, 61, 30, 50 mtrs turn radius b. Understeer gradient c. Characteristic speed d. Lateral acceleration gain at 96 kmph e. Yaw velocity gain at 96 kmph Load (Kg); Cornering stiffness (N/deg); Cornering coefficient (N/N/deg.). 102; 298; 0.298. 204; 539; 0.269. 306; 760; 0.253. 408; 1000; 0.250. 510; 1140; 0.228. 620; 1314; 0.222. | 10M |  |  | CO-5 | BL-4 |

***