| Subject Code 22PC1CE302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, December, 2024

WATER RESOURCES ENGINEERING

(CE)

Time: 3 hours                                                                         Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | What are the two common methods used to measure precipitation. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Differentiate between potential evapotranspiration and actual evapotranspiration. | 2M | CO-1 | BL-2 |
|  | What is the SCS-CN method? Mention its primary application. | 2M | CO-2 | BL-1 |
|  | Mention any two advantages and two ill effects of irrigation. | 2M | CO-3 | BL-1 |
|  | What is a command area in canal irrigation? | 2M | CO-4 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | Illustrate the Depth-Area-Duration (DAD) relationship, IDF curve with neat sketch and their importance in hydrologic analysis. | 10M | CO-1 | BL-2 |
|  | OR |  |  |  |
| 2. | a) Based on isopluvial maps the 50 year 24 – hour maximum rainfall at Bangalore is found to be 16.0 cm. Determine the probability of a 24 -h rainfall of magnitude 16.0 cm occurring at Bangalore: - Once in ten successive years - Twice in ten successive years - At least once in ten successive years. | 5M | CO-1 | BL-4 |
|  | b) Compare the various forms of precipitation and their significance in hydrology. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | a) A reservoir has a surface area of 200 km². If the depth of water in the Class A evaporation pan decreases by 3 cm from the initial level due to evaporation over a day. Estimate the volume of water lost from the reservoir due to evaporation. Assume a pan coefficient of 0.7. | 5M | CO-1 | BL-3 |
|  | b) Describe the infiltration indices and their applications in hydrologic analysis. | 5M | CO-1 | BL-2 |
|  | OR |  |  |  |
| 4. | a) A storm with 10cm of precipitation produced a direct runoff of 5.7cm. The duration of the rainfall was 16hours and its time distribution is given below. Estimate the - index of the storm. Time (hr); 0; 2; 4; 6; 8; 10; 12; 14; 16. Cumulative rainfall (cm); 0; 0.4; 1.3; 2.8; 5.1; 6.9; 8.5; 9.5; 10.0. | 5M | CO-1 | BL-3 |
|  | b) Discuss the concepts of interception and depression storage, including their impact on surface runoff. | 5M | CO-1 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5 | The ordinates of 3hr Unit Hydrograph are given below. Find the ordinates of 6hr Unit Hydrograph. Time; 0; 3; 6; 9; 12; 15; 18; 21; 24; 27; 30. Ordinates of 3hr U.H; 0; 10; 25; 20; 16; 12; 9; 7; 5; 3; 0. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 6. | a) In a 350ha watershed the CN value was assessed as 70 for AMC III. (a) Estimate the value of direct runoff volume for the following 4 days of rainfall. The AMC on July 1st was category III. Use standard SCS – CN equation. (b) What would be the runoff volume if CNIII value were 80. Date; July 1; July 2; July 3; July 4. rainfall (mm); 50; 20; 30; 18. | 5M | CO-2 | BL-3 |
|  | b) Explain the steps involved in base flow separation from a hydrograph. | 5M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) Discuss the cropping seasons in India and the major crops grown during each season. | 5M | CO-3 | BL-2 |
|  | b) Define the terms Duty and Delta and derive a relationship between them. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 8 | A water course has a culturable commanded area of 2600 hectares out of which the intensities of irrigation for perennial sugarcane and rice crops are 20% and 40% respectively. The duty for these crops at the head of water course are 250hect/cumec and 1800hect/cumec respectively. Find the discharge required at the head of water course if the peak demand is 120% of average requirement. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | a) Design an irrigation channel in alluvial soil according to Lacey’s silt theory for a discharge of 15m3/sec and silt factor f = 1.0 | 5M | CO-4 | BL-3 |
|  | b) Explain the classification of canal outlets with examples. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Design an irrigation channel to carry a discharge of 6 cumecs. Assume N=0.0225 and m=1. The channel has a bed slope of 0.25 m per kilometre. Use Kennedys theory. | 10M | CO-4 | BL-3 |

***