| Subject Code 22PC1ME305 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, May/June 2025

ADDITIVE MANUFACTURING

(ME)

Time: 3 hours                                                                                 		      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Discuss the significant applications of additive manufacturing in the aerospace industry. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What are the key components of an FDM machine? | 2M | CO-2 | BL-2 |
|  | Discuss about the photo polymerization involved in SLA with chain reaction sketch. ? | 2M | CO-2 | BL-2 |
|  | Does SLS typically require support structures? Why or why not? | 2M | CO-1 | BL-2 |
|  | What is the general purpose of post-processing in additive manufacturing? | 2M | CO-3 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Classify the Additive Manufacturing Processes with suitable block diagram. Discuss about the role of Additive Manufacturing technology in product development | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | What are the key advantages of using additive manufacturing over conventional methods? Consider aspects such as design freedom, customization, material efficiency, lead times, and cost-effectiveness. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Discuss the common challenges and limitations encountered in FDM. | 5M | CO-2 | BL-2 |
|  | - What are the specific design considerations for LOM? How does the layer-based cutting and bonding process impact the design of internal features and intricate shapes? | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Discuss about the working of FDM with a neat, labelled sketch along with the materials used. Mention its advantages and applications. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Discuss about the working of Binder Jetting with a neat, labeled sketch and discuss about the advantages and applications. ? | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6. | Eloborate the working principle and processes involved in Stereolithography (SLA) with a suitable sketch along with resins used and mention the advantages and applications involved in it. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Describe the fundamental principles and step-by-step working process of Selective Laser Sintering (SLS) with neat sketch and mention advantages and applications involved in it. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 8. | Describe the fundamental principles and step-by-step process of Electron Beam Melting. Explain the role of the electron beam, vacuum environment, and powder bed in creating a 3D metal object. | 10M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | Enumerate about the STL file format briefly and explain various problems involved in STL file with suitable sketches. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 10. | Explain about the various traditional inspection techniques and Non-Destructive Defect Detection Techniques that are involved in inspection of Additively Manufactured parts with suitable sketches. | 10M | CO-3 | BL-3 |

****