| Subject Code 22PE1EI306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, May/June 2025

BIO MEDICAL INSTRUMENTATION

(EIE)

Time: 3 hours                                                                                 		         Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5×2=10M

|  | List at least two applications of surface electrode. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Define systolic and diastolic pressure. | 2M | CO-2 | BL-2 |
|  | List different modes of operation in pacemaker. | 2M | CO-3 | BL-2 |
|  | What are the types of ventilators based on mode of operation? | 2M | CO-1 | BL-2 |
|  | Why is intraocular pressure (IOP) measurement important? | 2M | CO-4 | BL-4 |

PART-B					5×10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Distinguish between internal and external electrodes and give some examples and applications. | 5M | CO-1 | BL-4 |
|  | - Draw the reference electrode and explain its operation. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | a) List problems associated with measurements from living systems. | 3M | CO-1 | BL-2 |
|  | b) What are typical characteristics of a bio-amplifier? What are the advantages of using an instrumentation amplifier for bio-signals? | 7M | CO-2 | BL-4 |
| UNIT-II |  |  |  |  |  |
| 3. | - Write the principle and need for blood pressure measurement. | 5M | CO-2 | BL-2 |
|  | - Write the principle and need for blood flow measurement. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4. | Explain about cardiovascular system activities with diagrams as may be needed. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5. | - Write a brief note about neuronal communication. | 5M | CO-2 | BL-3 |
|  | - Draw the block diagram of EMG and give applications and limitations of EMG measurement. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 6. | What is Electroencephalogram? Explain the functioning of EEG system with the help of block diagram | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7. | - What is defibrillator? Explain the operation of synchronized DC defibrillators. | 5M | CO-3 | BL-3 |
|  | - Describe the operation of a pacemaker. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 8. | What is meant by dialysis? Describe the construction and operation of hemodialysis machine. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9. | - What is ERG? What are the electrodes used in ERG. Explain in brief. | 5M | CO-4 | BL-1 |
|  | - Describe the advantages and limitations of Computed Tomography scan. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Demonstrate the principle and working of MRI with neat block diagram. | 10M | CO-3 | BL-4 |

****