| Subject Code 22PC1CE306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, May/June 2025

ENVIRONMENTAL ENGINEERING

(CE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What are the objectives of water supply system? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List and write the function of unit operations in raw water treatment. | 2M | CO-2 | BL-2 |
|  | What is wet weather flow? List the factors affecting wet weather flow. | 2M | CO-3 | BL-1 |
|  | What is the function of screens in waste water treatment and what are different types of screens used in waste water treatment. | 2M | CO-3 | BL-2 |
|  | Draw oxygen sag curve and note different parameters on it. | 2M | CO-4 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) Discuss the necessity of protected water supply system to the public. | 5M | CO-1 | BL-1 |
|  | b)The population of a locality as obtained from census report is as follows: Census Yr.; 1981; 1991; 2001; 2011; 2021. Population; 350000; 4,66,000; 9,94,000; 15,00,000; 16,23,000. Estimate the population of the locality in the years 2031and 2041 by arithmetic increase method and geometrical increase method. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | Discuss the factors affecting selection of location for intake structure. Explain the functioning of river intake structure with neat sketch. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | a) Design a rectangular sedimentation tank to supply water for a population of 1, 00, 000 with average water supply of 135 lpcd. Assume detention period 3 hours and any required data. | 5M | CO-2 | BL-3 |
|  | b) Explain step by step procedure for back washing of rapid sand filters. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4. | a) What are the different methods of disinfection? Explain chlorination method of disinfection. | 5M | CO-2 | BL-1 |
|  | b) Describe the various layouts of distribution system with advantages and disadvantages. | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5. | a) Explain the factors affecting sanitary sewage generation? How do you calculate quantity of sanitary sewage? | 5M | CO-3 | BL-1 |
|  | b) Describe the sewer appurtenances. | 5M | CO-3 | BL-1 |
| OR |  |  |  |  |  |
| 6. | a) Design a combined sewer of a circular section to be laid to serve a particular area from the following data: Area to be served= 120 hectares, Population = 1, 00,000 Time of entry for storm water = 10 minutes, Coefficient of run-off = 0.45 Time of flow in channel =20 minutes, Per capita water supply= 250lpcd Maximum velocity of flow=3m/sec. Assume additional data, where necessary. | 5M | CO-3 | BL-3 |
|  | b) Explain the factors to be considered in selection of Sewer materials. | 5M | CO-3 | BL-1 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) Explain the functions of each unit of a domestic sewage treatment plant with a flow diagram. | 5M | CO-3 | BL-1 |
|  | b) Design a grit chamber for a maximum wastewater flow of 50 MLD, to remove particles up to 0.2 mm diameter having specific gravity 2.65. The kinematic viscosity of water is 1.141 X 10- 6 m2/s at 15°C. Assume additional data where necessary. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8. | a) Explain the working and construction of septic tank and soak pit. | 5M | CO-3 | BL-2 |
|  | b) Explain about sedimentation aided with coagulation process. | 5M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9. | a) Compare & contrast between trickling filter and activated sludge process. | 5M | CO-4 | BL-2 |
|  | b) Explain the construction and working of Oxidation pond. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10. | a) Discuss the self-purification process in water bodies. | 5M | CO-4 | BL-2 |
|  | b) What are the methods for disposal of sludge? Discuss about sludge drying. | 5M | CO-4 | BL-2 |

****