| Subject Code 22PC1ME304 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, May/June 2025

HEAT TRANSFER

(ME)

Time: 3 hours                                                                                 		      	     Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Note: Use of Heat and Mass Transfer Data Book is allowed.

PART-A					5X2=10M

|  | What is thermal diffusivity? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define the efficiency and effectiveness of a fin. | 2M | CO-1 | BL-1 |
|  | Distinguish between natural and forced convection heat transfer. | 2M | CO-2 | BL-2 |
|  | State the difference between sub-cooled and saturated boiling. | 2M | CO-2 | BL-1 |
|  | Define shape factor. | 2M | CO-3 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - State Fourier‘s law of heat conduction. Why is the negative sign used? | 5M | CO-1 | BL-2 |
|  | - The heat flow rate through a 3 cm thick wood board for a temperature difference of 300C between the surfaces is 120 W/m2. Calculate the thermal conductivity of Wood. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | Derive the general heat conduction equation in Cartesian coordinates and deduce it in one dimensional steady state condition with no internal heat generation. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | a) Take a composite slab with series and parallel resistances. Draw the thermal circuit representing temperatures, resistances and heat flow, in it. | 5M | CO-1 | BL-2 |
|  | b) What is the expression for critical radius of insulation in case of i) Cylinder ii) Sphere. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 4. | A 40×40 cm copper slab 5mm thick at a uniform temperature of 2500C suddenly has its surface temperature lowered to 300C. Find the time at which the slab temperature becomes 900C. Take ρ =9000kg/m3, Cp=380J/kg K, k = 370W/mK and h = 90W/m2K. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | Air at 100C and at atmosphere pressure flowing over a plate at a velocity of 3 m/sec. If the plate is 30 cm wide and at a temperature of 600C. Calculate the following quantities at x = 0.3m. (i) Hydrodynamic Boundary layer thickness (ii) Thermal boundary layer thickness (iii) Heat transfer coefficient (iv) Heat transfer rate from the plate. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 6. | Show by dimensional analysis that for Natural convection heat transfer, Nusselt number is a function of Grashof number and Prandtl number. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | Explain the pool boiling curve with a neat sketch. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 8. | For the capacity of 2500W, hot oil flows through a double pipe heat exchanger. It enters at 3600C and leaves at 3000C. Cold fluid enters at 300C and leaves at 2000C. If the overall heat transfer coefficient is 800 W/m2K, determine the heat exchanger area required for (i) Parallel flow (ii) Counter flow. | 10M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | - What is the Stefan-Boltzman Law? Explain the concept of emissivity. | 5M | CO-3 | BL-1 |
|  | - Explain the terms radiosity and irradiation. | 5M | CO-3 | BL-1 |
| OR |  |  |  |  |  |
| 10. | Consider radiative heat transfer between two large parallel plates of surface emissivities 0.8. If a thin radiation shield of emissivity 0.05 on both sides, is placed between surfaces to reduce the radiation heat transfer. How much will be the heat transfer reduction? | 10M | CO-3 | BL-3 |

****