| Subject Code 22PC1CE305 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, May/June 2025

SOIL MECHANICS

(CE)

Time: 3 hours                                                                                 		      	   Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Assume suitable data if necessary

PART-A

5X2=10M

|  | Give relation between void ratio and porosity. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Sketch the A-line chart used in USC system. | 2M | CO-2 | BL-1 |
|  | Write the validity of Darcy’s law. | 2M | CO-3 | BL-1 |
|  | Define compression index. | 2M | CO-4 | BL-1 |
|  | Define flow net. | 2M | CO-3 | BL-3 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) How will you determine water content of soil using oven-drying method? | 5M | CO-1 | BL-2 |
|  | b) Calculate the void ratio, porosity and degree of saturation of a soil sample if it is having wet density 2.0 g/cc and dry density 1.8 g/cc. specific gravity of soil is 2.7 | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
|  | a) How will you determine In-situ density of cohesive soil in the field | 6M | CO-1 | BL-2 |
|  | b) The soil in a borrow pit has a void ratio of 0.90. A fill-in-place volume of 20,000 m3 is to be constructed with an in-place dry density 18.84 kN/m3. If the owner of borrow area is to be compensated at Rs. 1.50 per cubic metre of the excavation, determine the cost of compensation. | 4M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
|  | Illustrate Indian standard classification system of soil with necessary chart. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | a) How will you determine liquid limit in the Laboratory and explain the result with an example graph. | 6M | CO-2 | BL-2 |
|  | b) A clay sample has a void ratio of 0.5 in dry condition. The grain specific gravity has been determined as 2.72. What will be the shrinkage limit of this clay? | 4M | CO-2 | BL -3 |
| UNIT- III |  |  |  |  |  |
|  | a) A soil sample 90 mm high and 6000 mm2 in cross-section was subjected to a falling head permeability test. The head fall from 500 mm to 300 mm in 1500 sec. The permeability of the soil was 2.4×10−3 mm/s. Determine the diameter of the stand pipe. | 5M | CO-3 | BL-3 |
|  | b) Provide suitable method for determining permeability of fine-grained soil and derive equation for determining k. | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |  |
|  | a) Determine the average horizontal and vertical permeabilities of a soil mass made up of three horizontal strata, each 1m thick, if the coefficients of permeability are 1 x 10-1 mm/s, 3 x 10-1 mm/s and 8 x 10-2 mm/s for the three layers. | 5M | CO-3 | BL-3 |
|  | b) Explain Flow nets, their Characteristics and Uses. | 5M | CO-3 | BL-3 |
| UNIT- IV |  |  |  |  |  |
|  | a) Sketch an ideal 'compaction curve' and discuss the effect of moisture on the dry unit weight of soil. | 5M | CO-4 | BL-2 |
|  | b) Explain why soils are compacted in the field. How is the degree of compaction ensured in the field? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
|  | a) In the consolidation test on a soil the void ratio of the sample decreased 1.25 to 1.10 when the pressure increased from 150 kN/m2 to 350 kN/m2. Calculate the coefficient of consolidation if the permeability is 7 x 10-7 cm/s. | 4M | CO-4 | BL-1 |
|  | b) An oedometer test is performed on a 3 cm thick clay sample. After 5 minutes, 50% consolidation is reached. After how long a time would the same degree of consolidation is achieved in the field where the clay layer is 5 m thick? Assume the sample and the clay layers have the same drainage boundary conditions (double drainage). | 6M | CO-4 | BL-3 |
| UNIT- V |  |  |  |  |  |
|  | a) Explain the total stress, neutral stress and effective stress. What is the role of effective stress in soil mechanics? | 5M | CO-3 | BL-3 |
|  | b) Explain the effect of water table fluctuations on effective stress. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
|  | a) Explain with a simple sketch, how direct shear test is conducted and how interpretation of results is done. | 5M | CO-4 | BL-2 |
|  | b) A cylindrical specimen of saturated soil fails under an axial stress 150 kN/m2 in an unconfined compression test. The failure plane makes an angle of 520 with the horizontal. Calculate the cohesion and angle of internal friction of the soil. | 5M | CO-4 | BL-3 |

***