| Subject Code 22PE1CE304 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, May/June 2025

TRAFFIC ENGINEERING

(CE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Differentiate between time mean speed and space mean speed. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is meant by level of service (LOS)? | 2M | CO-2 | BL-2 |
|  | Mention two problems caused by on-street parking. | 2M | CO-3 | BL-3 |
|  | What is a simple progression system in traffic signal coordination? | 2M | CO-3 | BL-3 |
|  | Name two air pollutants emitted by vehicles. | 2M | CO-4 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Describe the various methods for conducting speed studies and mention how the data is presented. | 5M | CO-1 | BL-2 |
|  | - During a 5-minute interval, 120 vehicles passed a point. The average headway is observed to be 2.5 seconds. Calculate: (a) The flow in vehicles/hr (b) Average space headway if average speed is 36 kmph | 5M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
| 2. | Derive the relationship between flow, speed, and concentration. Plot the fundamental diagram and explain the relationships (Q-K, Q-V, K-V). | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | - Discuss the significance of service volume and service flow rate. | 5M | CO-2 | BL-2 |
|  | b) A 6-lane divided multilane highway has a base capacity of 2200 vehicles/hour/lane. Due to lane width being less than ideal and presence of lateral obstructions, the adjustment factor is 0.88. Compute:(a) Adjusted capacity per lane(b) Total adjusted capacity for the full highway section | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 4. | Describe the steps involved in the computation of capacity and LOS for: (a) Two-lane highway (b) Multilane highway | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5. | - Describe the purpose and steps of conducting a road safety audit. | 5M | CO-3 | BL-2 |
|  | - Discuss the various types of parking facilities with neat sketches. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | A parking survey conducted in a shopping area for 5 hours showed the following data: a)Total number of parking spaces: 50 b)Total number of vehicles parked during the study: 180 c)Total parking duration by all vehicles: 375 vehicle-hours Calculate: (a) Parking accumulation (b) Parking turnover (c) Parking load (d) Parking efficiency | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Explain the objectives of installing traffic signals at an intersection. | 5M | CO-3 | BL-2 |
|  | - Explain the different types of traffic signals. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 8. | At a four-leg intersection, the observed critical flow ratios on the approaches are: a)Approach A: 0.30 b)Approach B: 0.25 c)Approach C: 0.20 d)Approach D: 0.15 Lost time per phase is 4 seconds. Calculate: (a) Optimum cycle length using Webster’s formula (b) Effective green time for each phase assuming proportional allocation | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9. | - Explain any five detrimental effects of traffic on the environment. | 5M | CO-4 | BL-1 |
|  | - What is the impact of traffic congestion on Air and Noise pollution. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Discuss the various technological and policy-level interventions to reduce vehicular air pollution. Provide suitable examples. | 10M | CO-4 | BL-2 |

****