| Subject Code A19PE1AE10 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December, 2024

AUTOMOTIVE CONTROL SYSTEMS

(AE)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define Closed loop control system. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define time response of a system. | 2M | CO-2 | BL-1 |
|  | What is the purpose of a Bode plot? | 2M | CO-2 | BL-1 |
|  | What is meant by absolute stability? | 2M | CO-3 | BL-1 |
|  | Define root locus. | 2M | CO-3 | BL-1 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Derive the transfer function of AC server motor. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | Determined the transfer function C(s)/R(s) of the system shown in Fig. | 10M | CO-1 | BL-3 |
| UNIT-2 |  |  |  |  |
|  | Analyse the time response of a second-order system to a unit step input, including concepts of overshoot, settling time, and rise time. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | Compare P, PI, and PID control modes in terms of their effects on transient and steady-state responses. | 10M | CO-2 | BL-2 |
| UNIT-3 |  |  |  |  |
|  | Explain in detail the frequency-domain specifications: peak resonance, resonant frequency, bandwidth, and cut-off rate with their significance in system analysis. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | Determine the band width, resonant peak, GM and PM for the system with G(s)=4s(s+1). Assume H(s)=1. | 10M | CO-2 | BL-3 |
| UNIT-4 |  |  |  |  |
|  | The characteristic equation of a control system is given by s4+20s3+15s2+2s+K=0. Use Routh-Harwitz method to find the value of K for which the system will be marginally stable and the frequency of the corresponding sustained oscillations. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Draw the Bode plot for the system having GsHs=100(0.02s+1)s+10.1s+10.01s+1 Find the gain and phase cross over frequency. | 10M | CO-3 | BL-3 |
| UNIT-5 |  |  |  |  |
|  | The open loop transfer function of the control system is given by G(s)H(s)=K[s(s+6)(s2+4s+13)], sketch the root locus. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Derive the transfer function of a lag-lead compensator and discuss its characteristics. | 10M | CO-3 | BL-2 |
| UNIT-6 |  |  |  |  |
|  | Obtain the state space representation of the electrical network shown in Fig. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | A system is described by X=-1-4-1-1-6-2-1-2-3 X1X2X3+011 U and Y=111 X1X2X3. Find the transfer function. | 10M | CO-4 | BL-3 |

***