| Subject Code A19PE1CE11 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December, 2024

FOUNDATION ENGINEERING

(CE)

Time: 3 hours                                                                                 	            	        Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

Assume the data suitably, if required.

PART-A

5X2=10M

|  | Determine the area ratio for a shelby tube sampler with external diameter of 51mm and internal diameter 48mm. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Mention different types of slope failures. | 2M | CO-2 | BL-1 |
|  | State the assumptions in Rankine’s earth pressure theory. | 2M | CO-3 | BL-1 |
|  | List out the different types of retaining walls. | 2M | CO-3 | BL-1 |
|  | Define negative skin friction. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Discuss standard penetration test. What are the various corrections? Write the importance of the test in Geotechnical Engineering. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Enumerate different types of Samplers. Explain any one in detail. | 5M | CO-1 | BL-3 |
|  | b) Explain in detail electrical resistivity method. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
|  | Explain Swedish Circle method in detail with neat sketch. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
|  | a) If an embankment of 10 m height is to be made from a soil having c' = 25 kN/m2, φ' = 15°, and γ=18 kN/m3, what will be the safe angle of slope for a factor of safety of 1.5? | 5M | CO-4 | BL-3 |
|  | b) What is Taylor’s stability number? How it is useful in slope analysis? | 5M | CO-4 | BL-2 |
| UNIT-III |  |  |  |  |  |
|  | A rigid retaining wall is subjected to passive earth pressure. Determine the passive earth pressure distribution and the magnitude and point of application of the resultant thrust by Rankine theory. Given: Height of wall = 10 m; depth of water table from ground surface = 3 m; c=20 kN/m2, φ = 20° and γsat = 19.5 kN/m3. The backfill carries a uniform surcharge of 20 kN/m2. Assume the soil above the water table is saturated. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
|  | a) Differentiate between Rankine’s and Coulomb’s theories of lateral earth pressures. | 5M | CO-3 | BL-2 |
|  | b) An 8 m high retaining wall is supporting a c-φ backfill having c = 40kN/m2; φ = 24°; γ = 18.50 kN/m3. Plot the distribution of active and passive earth pressure and determine the magnitude and point of application of total active and passive earth pressure acting on the retaining wall after the generation of crack if any. | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
|  | Discuss different types of retaining walls in detail with neat sketches. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Explain the steps involved in design of a retaining wall. | 5M | CO-2 | BL-3 |
|  | b) Summarize the drainage characteristics of backfill in retaining wall. | 5M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |  |
|  | Explain in detail plate load test with a neat diagram. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Explain general, local and punching shear failure with neat figures. | 5M | CO-3 | BL-2 |
|  | b) A square foundation is 2m x 2m in plan. The soil supporting the foundation has friction angle of φ = 25° and c’ = 20 kN/m2. The unit weight of soil γ is 16.5 kN/m3. Determine the allowable gross load on the foundation with a factor of safety of 3. Assume the Df is 1.5m and that general shear failure occurs in the soil. | 5M | CO-3 | BL-3 |
| UNIT-VI |  |  |  |  |  |
|  | What are different types of piles and their functions? Explain with sketches. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
|  | a) A square group of 9 piles was driven into soft clay extending to a large depth. The diameter and length of the piles were 30 cm and 9 m respectively. If the unconfined compression strength of the clay is 90kN/m2, and the pile spacing is 90 cm centre to centre, what is the capacity of the group? Assume a factor of safety of 2.5 and adhesion factor of 0.75. | 5M | CO-3 | BL-3 |
|  | b) How do you estimate safe load carrying capacity from the results of pile load test? | 5M | CO-3 | BL-2 |

***