| Subject Code A19PE1ME12 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December, 2024

MECHANICS OF COMPOSITE MATERIALS

(ME)

Time: 3 hours                                                        		         			     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | What are the major constituents in composite materials? | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | Expand the following laminate code - [0/45/30]S ii) [0/90]2 | 2M | CO1 | BL2 |
|  | Define Orthotropic material and give the number of elastic constants in macro mechanics. | 2M | CO3 | BL1 |
|  | Distinguish between symmetric cross ply laminate and symmetric angle ply laminate. | 2M | CO3 | BL2 |
|  | What is meant by knitting. | 2M | CO2 | BL1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | What is a composite material? Give the classification of composite materials. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Explain about the role and selection of Matrix in composite materials. b) Discuss about different types of matrix materials used in composite materials. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |  |
|  | Discuss various types of reinforcements used in composite materials | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Calculate the longitudinal modulus and tensile strength of a unidirectional composite containing 55 percent by volume of Sisal fibers in epoxy matrix. The modulus and strength of fiber is 30 GPa and 600 MPa respectively and the same for matrix is 3.5 GPa and 100 MPa respectively. Find the fraction of load taken by fibers in the composite. b) State the assumptions in applying Iso-strain and Iso-stress conditions while determining longitudinal and transverse properties. | 6M 4M | CO-1 CO-1 | BL-3 BL-2 |
| UNIT-III |  |  |  |  |  |
|  | Explain the manufacturing process of Metal matrix composites with Solid state diffusion technique with neat sketch and list the applications of metal matrix composites. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Explain Stir casting process of manufacturing metal matrix composite with neat sketch b) Explain the properties and applications of carbon -carbon composites | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-IV |  |  |  |  |  |
|  | Explain in detail about Filament winding method of manufacturing polymer matrix composites. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Why is it easier to process with thermosets than with thermoplastics b) Explain about the characterization of polymer matrix composite | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-V |  |  |  |  |  |
|  | Determine the stiffness and compliance matrices for a unidirectional lamina that has the following elastic constants: EL= 20 GPa, ET =2 GPa, GLT = 0.7 GPa, ʋLT = 0.35 | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
|  | Calculate Ex, Gxy, ʋxy,mx and my at 300,450 and 600 for an orthotropic lamina having the following properties.EL = 14 GPa, ET = 3.5 GPa, GLT = 4.2 GPa, ʋLT =0.4 | 10M | CO-3 | BL-3 |
| UNIT-VI |  |  |  |  |  |
|  | Longitudinal axis of an orthotropic lamina makes an angle of 450 with the x axis. It is subjected to the following stresses: x = 20 MPa, y = 0,xy = 20 MPa, LU= 500 MPa, TU= 10 MPa,'LU= 350 MPa, 'TU= 75 MPa, LTU = 35 MPa. Does the lamina fail. Use the maximum work theory to predict the failure | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
|  | Explain about different failure theories applicable for composite material. | 10M | CO-4 | BL-3 |

***