| Subject Code A19PC1EC13 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December, 2024

MICROWAVE ENGINEERING

(ECE)

Time: 3 hours                                                                                 		      Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Write the relationship between phase and group velocities. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | List different types of attenuators. | 2M | CO-1 | BL-2 |
|  | Explain the principle of Isolator. | 2M | CO-2 | BL-2 |
|  | Differentiate between two cavity klystron and reflex klystron. | 2M | CO-3 | BL-3 |
|  | Write the applications of PIN diode. | 2M | CO-3 | BL-2 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Starting with the equation for the propagation constant of a mode in rectangular wave guide, Derive the expression. λ=λgλcλg2+λc2 | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | a) Show that the TEM, TM01 and TM10 modes in a rectangular wave-guide do not exist. b) A rectangular wave guide with dimension of 3 × 2 cm operates in the TM11Mode at 10 GHz. Determine the characteristic wave impedance. | 5M 5M | CO-1 CO-1 | BL-3 BL-3 |
| UNIT-II |  |  |  |  |
|  | How many types of waveguide phase shifters are there? Discuss one type in detail. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | a) Write short notes about probes and loops. b) Explain about Flap attenuator. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-III |  |  |  |  |
|  | With a neat sketch explain the construction and operation of H-plane Tee junction and derive its S matrix. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | a) State and explain the properties of S parameters. b) In an H-plane Tee junction, 20mW power is applied to port 3 that is perfectly matched to the junction. Calculate the power delivered to the load 60Ω and 75Ω connected to ports 1 and 2. | 5M 5M | CO-2 CO-2 | BL-3 BL-3 |
| UNIT-IV |  |  |  |  |
|  | With neat sketches, explain the operation of 8 cavity cylindrical magnetron. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | a) What is velocity modulation? Explain velocity modulation with necessary sketches. b) Explain about two cavity Klystron structure. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-V |  |  |  |  |
|  | Explain the operation of IMPATT diode. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | a) Discuss the merits and demerits of IMPATT diode. b) Explain the Gunn diode with its Gunn oscillation modes. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-VI |  |  |  |  |
|  | Draw the microwave bench setup to measure the attenuation of given fixed attenuator and explain the measurement procedure. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | a) Give the block diagram for the measurement of impedance at microwave frequencies and explain the procedure. b) Calculate the SWR of a transmission system operating at 8GHz. The distance between two minimum power points is 0.9mm on a slotted line whose velocity factor is unity. | 5M 5M | CO-4 CO-4 | BL-2 BL-3 |

***