| Subject Code A19PC1EC30 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December 2024

PRINCIPLES OF DIGITAL SIGNAL PROCESSING

(EEE)

Time: 3 hours                                                                                 		      		Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Determine whether the following signal is periodic or not. If a signal is periodic, specify its fundamental period. 𝑥( 𝑛) = 2 𝑐𝑜𝑠( π3n) + 𝑠𝑖𝑛 (3π4n) | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain how to find linear convolution of two sequences using DFT. | 2M | CO-2 | BL-2 |
|  | Find the Z-transform of 𝑥(𝑛) = 2n 𝑢( 𝑛 – 2) | 2M | CO-3 | BL-2 |
|  | What are the requirements for converting stable analog filter into a stable digital filter | 2M | CO-3 | BL-2 |
|  | Mention the difference between Harward and super harward architectures. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Explain about sampling theorem and find Nyquist sampling rate for given xn=4cos4π3n+2sinπ3n | 10M | CO-1 | BL-1 |
| OR |  |  |  |  |  |
|  | Define discrete time system? Explain the following systems with an example: a) Linear and non-linear systems. b) Stable and unstable systems. | 5M 5M | CO-1 CO-1 | BL-1 BL-1 |
| UNIT-II |  |  |  |  |  |
|  | Find IDFT of the given sequence using DFT X(k)={4, J2 ,0 ,-J2} | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | Find 8-point DFT of the sequence 𝑥 (𝑛) = [2, 2, 2, 2, 1, 1, 1, 1] using radix-2 DIF-FFT algorithm. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
|  | Find the Z-Transform of the given system represented by the difference equation 𝑦(𝑛) + 2 𝑦(𝑛 − 1) + 𝑦(𝑛 − 2) = 𝑥(𝑛) + 𝑥(𝑛 − 1) | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Find the z-transform of the signal, 𝑥(𝑛) = 12n 𝑢(𝑛) b) What are the advantages of Z-transform? Determine inverse Z-Transform of XZ=11+z-1(1-z-1) for ROC : Z >1. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-IV |  |  |  |  |  |
|  | Design an analog Butterworth filter that has a -2db pass band attenuation at a frequency of 0.2π rad/sec and at least -10dB stop band attenuation at 0.5π rad/sec, using bilinear transformation. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
|  | Realize given IIR filter transfer function in Direct form-II and cascaded structures XZ=1+13z-11-34z-1+18z-2 | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
|  | The desired response of a low-pass filter is Hdejω=e-3jω, &-3π4≤ω≤3π40 , other values Design FIR filter of order N = 7 using a Hamming window. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
|  | a) Explain the FIR filter design using windowing technique b) Compare FIR and IIR filters | 5M 5M | CO-3 CO-3 | BL-2 BL-1 |
| UNIT-VI |  |  |  |  |  |
|  | Draw and explain the internal architecture of TMS320C67X processor? | 10M | CO-4 | BL-1 |
| OR |  |  |  |  |  |
|  | Explain the generation of PWM Signals Using DSP Processor | 10M | CO-4 | BL-2 |

***