| Subject Code A19OE1EC09 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2025

EMBEDDED SYSTEMS FOR IOT

(Open Elective)

Time: 3 hours                                                                                 		      	 Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Discuss the characteristics of Embedded Systems. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | State the purpose of filtering in sensor systems? Distinguish between low-pass and high-pass filters. | 2M | CO-2 | BL-1 |
|  | What is the clock speed of a typical Arduino Uno board? Name any two communication interfaces available on an Arduino board. | 2M | CO-3 | BL-1 |
|  | What is the difference between IoT and M2M? | 2M | CO-3 | BL-1 |
|  | List any two IoT devices commonly used in industrial automation. How does IoT contribute to energy efficiency? | 2M | CO-4 | BL-1 |

PART-B					6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | What is system integration in embedded systems? State the role of simulation in embedded system integration? | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
| 2. | Illustrate the challenges in Embedded System Design. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |  |
| 3. | Discuss the key parameters that determine the performance of ADCs in smart sensors. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | Illustrate the architecture of a typical smart sensor system and explain. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5. | Brief out various types of sensors used in smart home appliances and their functions. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 6. | a) Discuss the role of wireless sensor networks in large-scale environmental monitoring. | 3M | CO-2 | BL-2 |
|  | b) Draw and explain with a neat block diagram, how environmental hazards are spread. | 7M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7. | Describe the steps involved in creating a WiFi-controlled robot car using Arduino. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8. | Explain the interfacing of 7-segment display to Arduino with a neat sketch. | 10M | CO-1 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | Analyze the components of physical and logical design in an IoT system. How do they complement each other to enable IoT functionality? | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 10. | Explain in detail the four communication models of IoT. | 10M | CO-2 | BL-1 |
| UNIT-VI |  |  |  |  |  |
| 11. | Describe the implementation of IoT in environmental monitoring and its impact on sustainability. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 12. | Discuss in detail the steps involved in IoT system design methodology. | 10M | CO-4 | BL-2 |

***