| Subject Code A19PE1EE13 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2025

HVDC TRANSMISSION

(EEE)

Time: 3 hours                                                                                 		      	 Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A 				5X2=10M

|  | What are the applications of DC Transmission | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the advantages of LCC in HVDC applications? | 2M | CO-2 | BL-1 |
|  | Why is current control essential in HVDC systems? | 2M | CO-2 | BL-1 |
|  | What is a Multi-Terminal DC (MTDC) system? | 2M | CO-3 | BL-1 |
|  | What are the types of AC filters? | 2M | CO-4 | BL-1 |

PART-B  			 	6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Explain the historical development in DC Transmission? | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | a) Give the comparison between AC and DC Transmission and explain any two factors in detail? | 5M | CO-1 | BL-2 |
|  | b) What is a homo-polar HVDC transmission line? | 5M | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |
|  | Explain the analysis of Graetz circuit and waveforms for overlap angle. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | a) Explain the characteristics harmonics and non-characteristics harmonics. | 5M | CO-2 | BL-2 |
|  | b) Explain the rectifier and inverter modes of operation in HVDC system? | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
|  | Explain how to control the power in the HVDC link. What is a drawback in power control? | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | a) Discuss equidistant pulse firing angle control scheme with its relative merits and demerits | 5M | CO-2 | BL-2 |
|  | b) Explain Starting and Stopping of DC-link? | 5M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
|  | Explain the types of MTDC systems and compare them. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | a) Explain how does an MTDC system differ from a traditional point-to-point HVDC system? | 5M | CO-3 | BL-2 |
|  | b) Explain the potential application of MTDC system? | 5M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
|  | Obtain a mathematical model of DC link. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | a) Explain Why power flow analysis is essential for planning and operating HVDC networks? | 5M | CO-4 | BL-2 |
|  | b) What are the key equations governing a DC transmission network | 5M | CO-4 | BL-2 |
| UNIT-VI |  |  |  |  |
|  | Explain over voltage and current protection in the converter station? | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | a) Explain the Design procedure of AC Filter? | 5M | CO-4 | BL-2 |
|  | b) Explain the Surge arrester and smoothing reactor in detail? | 5M | CO-4 | BL-2 |

***