| Subject Code A19PE1CE21 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2025

PAVMENT ANALYSIS AND DESIGN

(CE)

Time: 3 hours                                                                                 		      	 Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A				5X2=10M

|  | Differentiate between tyre pressure and contact pressure. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What are the assumptions of 2 layer theory? | 2M | CO-2 | BL-1 |
|  | What are the Westrgaard’s assumptions for rigid pavements? | 2M | CO-2 | BL-1 |
|  | Differentiate between rutting and fatigue. | 2M | CO-3 | BL-2 |
|  | List out the various types of overlays. | 2M | CO-4 | BL-1 |

PART-B				6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Explain the functions of various components of flexible and rigid pavements with neat sketches. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | - How do you calculate the ESWL at a given depth below the pavement for a dual wheel assembly? | 5M | CO-1 | BL-2 |
|  | - Briefly explain the various factors to be considered for in the design of flexible pavements? | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
|  | - Define a layered elastic system. Why is it used in flexible pavement analysis? | 5M | CO-2 | BL-2 |
|  | - Discuss the concept of single layer theory for flexible pavement. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | Explain the concept of viscoelastic theory for pavement. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
|  | Calculate the wheel load stresses at interior, edge and corner regions of a concrete pavement using Westergaard’s stress equation for the following data: Wheel load=5100 kg, Tyre Pressure=7 kg/cm2 Modulus of elasticity of concrete=3.0x105 kg/cm2 Pavement thickness=18cm, Modulus of subgrade reaction=6.0kg/cm3 Diameter of loaded area =15cm, Poisson’s ratio of concrete= 0.15 | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | - Discuss the use of ‘dowel bars’ and ‘tie bars’ in rigid pavements. | 5M | CO-2 | BL-2 |
|  | - Explain the significance of temperature stresses in rigid pavement design. | 5M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
|  | Explain the design of flexible pavements as per IRC recommendations. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | a) The initial traffic after completion of a four-lane divided highway is estimated as 3000cvpd.Design the flexible pavement for a life of 15 years using the following data.Design CBR = 8%, Growth rate of commercial vehicles = 7.5% , Average VDF = 4.0 | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
|  | Explain briefly the calibrated mechanistic design process for rigid pavements as per IRC. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | - What are the different types of joints in Rigid pavement. | 5M | CO-3 | BL-3 |
|  | - How do you calculate the length and spacing of tie bars? | 5M | CO-3 | BL-3 |
| UNIT-VI |  |  |  |  |
|  | Enumerate step by step procedure for overlay design by Benkelman beam deflection studies with neat sketch. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | Benkelman beam studies were carried out on 15 selected points on a stretch of a flexible pavement during summer season using a dual wheel load of 4085kg and 5.6kg/cm2 pressure. The deflection values obtained in mm after making the necessary lag correction are given below. If the present traffic consists of 750commercial vehicles per day, determine the thickness of the bituminous overlay required, if the pavement temperature during the test was 36° C and the correction factor for subsequent increase in subgrade moisture content is 1.3. Assume annual growth rate of traffic as 7.5%. Adopt IRC guidelines. 1.40, 1.32, 1,25, 1.35, 1.48,1.60, 1.65, 1.55, 1.45, 1.40, 1.36, 1.46, 1.50, 1.52, 1.45. Assume number of years before construction of pavement is 2. Adopt allowable deflection as1.0mm and equivalent factor for bituminous pavement as 2. | 10M | CO-4 | BL-4 |

***