| Subject Code 25BS1PH102 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year I Semester Regular Examinations, January 2026

ADVANCED ENGINEERING PHYSICS

(Common to EEE, ECE, CSE-AIML, CSE-IoT and EVL)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | List the differences between ordinary light and laser. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | State the Heisenberg uncertainty principle. | 2M | CO-2 | BL-1 |
|  | The energy band gap of a semiconducting material is 1.46 eV; find the wavelength of the LED source. | 2M | CO-3 | BL-1 |
|  | Classify magnetic materials. | 2M | CO-4 | BL-2 |
|  | Define qubit. | 2M | CO-5 | BL-1 |

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Develop the expression for the numerical aperture of an optical fiber. Recall the various applications of an optical fiber. | 7M | CO-1 | BL-3 |
|  | - For a given fiber with the refractive indices of the core and cladding are found to be 1.5 and 1.45, respectively. Find the acceptance angle. | 3M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 2 | a) Explain the construction and working of the He-Ne gas laser with a neat sketch. | 7M | CO-1 | BL-2 |
|  | b) Summarize Einstein coefficients and population inversion. | 3M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | - Build Schrödinger’s time-independent equation. Write the significance of the wave function. | 8M | CO-2 | BL-3 |
|  | - Estimate the ground state energy for an electron (mass 9.1×10-31 kg, h=6.626×10-34 J s) in the 1D box of length 1 nm. | 2M | CO-2 | BL-5 |
| OR |  |  |  |  |
| 4 | a) Analyze the formation of allowed energy bands and forbidden energy bands based on the Kronig-Penney model. | 8M | CO-2 | BL-4 |
|  | b) Sketch conductors, semiconductors, and insulators based on the energy level diagram. | 2M | CO-2 | BL-1 |
| UNIT-III |  |  |  |  |
| 5 | - State Hall Effect. Explain the phenomenon of the Hall Effect in semiconductors and derive the expression for the Hall coefficient. | 7M | CO-3 | BL-2 |
|  | - Distinguish between direct and indirect band gap semiconductors. | 3M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 6 | - Construct the ball milling method to synthesize nano materials | 5M | CO-3 | BL-3 |
|  | - Identify the average crystallite size using Debye-Scherrer’s formula. | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Discuss polarization types in dielectrics. | 7M | CO-4 | BL-3 |
|  | - List the various applications of dielectric materials in daily life. | 3M | CO-4 | BL-1 |
| OR |  |  |  |  |
| 8 | - Describe the hysteresis loop of ferromagnetic materials with a neat diagram | 6M | CO-4 | BL-2 |
|  | - Summarize the application of magnetic hyperthermia in cancer treatment within the field of medicine. | 4M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Explain quantum gates and quantum entanglement. | 6M | CO-5 | BL-2 |
|  | - Discuss by drawing Bloch's sphere. | 4M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10 | Discuss the challenges and advantages of quantum computing over classical computation. | 10M | CO-5 | BL-2 |

****