| Subject Code 25BS1CH101 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year I Semester Regular Examinations, January 2026

CHEMISTRY FOR ENGINEERS

(Common to CSE, EIE, IT, R&AI, CSE-CyS, CSE-DS and AI&DS)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Why hardness is expressed in CaCO3 equivalents? Mention the formula of CaCO3 equivalents. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Differentiate galvanic corrosion and pitting corrosion? | 2M | CO-2 | BL-4 |
|  | Distinguish between a primary battery and a secondary battery. | 2M | CO-3 | BL-4 |
|  | Specify which of the following polymers are condensation or addition polymers. - Teflon - Kevlar - Buna-S - Polyethylene | 2M | CO-4 | BL-3 |
|  | Write any two characteristic features of biosensing materials | 2M | CO-5 | BL-2 |

PART- B [5X10=50 Marks]

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | a) A sample of hard water was collected from Bala Nagar area and found to contain the following dissolved salts per litre. Ca(HCO3)2 = 18.2 mgs, MgCO3 = 35.6 mg/L, CaCl2= 17mgs, MgSO4=17.6mgs, suspended matter =10 mgs. Calculate the temporary, permanent, and total hardness of water in ppm. | 5M | CO-1 | BL-4 |
|  | b) Explain how scale formation and sludge formation take place in boilers. How are they removed? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2 | What is meant by desalination? How is reverse osmosis helpful in treating water, explain with a neat diagram. Compare reverse osmosis with the ion exchange method. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | a) Explain about the construction and working of the Standard Hydrogen Electrode (SHE). | 5M | CO-2 | BL-2 |
|  | b) Why does an iron screw rust faster in copper parts? With chemical reactions explain the mechanism of such corrosion. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4 | How cathodic protection alters the process of corrosion of a metallic structure, explain about different methods of cathodic protection. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5 | a) How to construct a proton exchange membrane fuel cell, mention the reactions of such a cell at each electrode, and list out applications | 5M | CO-3 | BL-3 |
|  | b) What is Syngas? How Fischer-Tropsch process is useful in synthesis of fuels, explain. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6 | - Explain the composition, salient characteristics, and uses of LPG, CNG, and Hythane. - How is green hydrogen generated by using water electrolysis? Explain. | 5M 5M | CO-5 CO-5 | BL-2 BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7 | a) Write the synthesis, properties, and applications of Kevlar. | 5M | CO-4 | BL-2 |
|  | b) Differentiate between addition and condensation polymers | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 8 | What does "Shape memory polymer" mean? Describe their classification and applications. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | a) Demonstrate the interpretative applications of UV–visible spectroscopy in detecting and analysing pollutants of the dye industry. | 5M | CO-5 | BL-3 |
|  | b) Describe the role of IR spectroscopy in night vision—security and pollution under control—CO sensor. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |  |
| 10 | - Explain the working principle and applications of STM - What are self-healing materials? Enumerate their working principle and applications. | 5M 5M | CO-5 CO-5 | BL-2 BL-2 |

****