| Subject Code 25ES1CE101 |  |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

APPLIED MECHANICS

(CE)

Time: 3 hours							          			Max. Marks: 60

Answer ALL Questions from PART-A

Answer ONE question from each unit of PART-B

PART-A					5X2=10M

| a. | State and explain parallelogram law of forces. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
| b. | Sketch the different types of supports and show their support reactions. | 2M | CO-1 | BL-2 |
| c. | Define the following: (i) Angle of Friction (ii) Coefficient of Friction | 2M | CO-2 | BL-1 |
| d. | Define the following terms. (i) Polar Moment of Inertia (ii) Radius of Gyration | 2M | CO-3 | BL-1 |
| e. | Define static indeterminacy. How is the degree of static indeterminacy determined for a frame? | 2M | CO-4 | BL-1 |

PART-B			    	           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - State and explain the Principle of Superposition and the Principle of transmissibility of forces. | 5M | CO-1 | BL-2 |
|  | - A horizontal line PQRS is 12 m long, where PQ = QR = RS = 4 m. Forces of 1000 N, 1500 N, 1000 N and 500 N act at P, Q, R and S respectively with downward direction. The lines of action of these forces make angles of 90°, 60°, 45° and 30° respectively with PS in anti-clock wise direction. Find the magnitude, direction and position of the resultant force. | 5M | CO-1 | BL-2 |
| (OR) |  |  |  |  |
| 2. | A system of connected flexible cable shown in Fig.1 is supporting two vertical forces 200 N and 250 N at points B and D. Determine the forces in various segments of the cable. Fig. 1 | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | Two parallel 40 N forces are applied to a lever as shown in Fig.2. Determine the moment of the couple formed by the two forces: - by resolving each force into horizontal and vertical components and adding the moments of the two resulting couples. - by using the perpendicular distance between the two forces. - by summing the moments of the two forces about point A.. Fig. 2 | 10M | CO-1 | BL-3 |
| (OR) |  |  |  |  |
| 4. | Determine the reactions at the supports of a simply supported beam AB shown in Fig. 3. Fig. 3 | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - State laws of friction. | 4M | CO-2 | BL-1 |
|  | - A body resting on a rough horizontal plane, required a pull of 180 N inclined at 30° to the plane just to move it. It was found that a push of 220 N inclined at 30° to the plane just moved the body. Determine the weight of the body and the coefficient of friction. | 6M | CO-2 | BL-3 |
| (OR) |  |  |  |  |
| 6. | Find the centroid of the shaded area shown in Fig.4 with reference to the X-Y coordinate system. Fig. 4 | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Derive an equation for moment of inertia of the following sections about centroidal axis: a) A rectangular section b) A triangular section from its base | 10M | CO-3 | BL-2 |
| (OR) |  |  |  |  |
| 8. | A built up section is made of two channel sections and two rectangular plates as shown in Fig. 5. Determine the moment of inertia of the built-up section about X-X axis passing through its centroid. Fig. 5 | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | A truss of span 10 meters is loaded as shown in Fig.6. Find the forces in all the members of the truss. Use method of joints. Fig. 6 | 10M | CO-4 | BL-4 |
| (OR) |  |  |  |  |
| 10. | - Determine the force in the members GE, GC, and BC of the truss shown in Fig.7. Indicate whether the members are in tension or compression. Use method of sections. Fig. 7 | 6M | CO-4 | BL-4 |
|  | - What is a cantilever truss? How will you find out the forces in the members of a cantilever truss. | 4M | CO-4 | BL-2 |

****