| Subject Code 25ES1EE102 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

Basic Electrical and Electronic Engineering

(Common to CS-AIML & CS-IoT)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Give the statement of Thevenin’s and superposition theorems. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define the terms average and RMS value of alternating component. | 2M | CO-2 | BL-2 |
|  | List the applications of synchronous and stepper motors. | 2M | CO-3 | BL-1 |
|  | Give the fundamental difference between PN junction diode and Zener diode. | 2M | CO-4 | BL-2 |
|  | List the applications of operational amplifier. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Find the current flowing through the terminals ‘a’ and ‘b’ | 5M | CO-1 | BL-3 |
|  | - A network consists of star type connection with all equal value of 4 Ω resistors of find its corresponding equivalent delta type connection. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2 | - Using superposition theorem find the current I flowing through the circuit. | 5M | CO-1 | BL-3 |
|  | - With considering a suitable example explain Thevenin’s theorem. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | - Find the form factor and peak factor of a sinusoidal alternating waveform with appropriate derivations. | 5M | CO-2 | BL-3 |
|  | - Derive equations for voltage and currents of 3 phase delta connections. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4 | A 30 Ω resistance, 40 mH inductance and 10 µF capacitance are connected in series and the circuit is fed from a 230 V, 50 Hz AC supply. Find voltage across the resistor, inductor, capacitor, real, reactive and apparent powers. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | - Discuss the working principle of DC motor and derive an equation for its back emf. | 5M | CO-3 | BL-3 |
|  | - Explain the torque-slip characteristics of 3 phase induction motor. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6 | Illustrate the working principles of synchronous generator, synchronous motor. Explain the significance of synchronous speed in their operation. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7 | - With the necessary circuit diagram discuss the functional operation of center tap full wave rectifier. | 6M | CO-4 | BL-3 |
|  | - Compare the half wave and full wave rectifiers. | 4M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 8 | Illustrate the basic operational details of ‘npn’ transistor. Draw the input, output V-I characteristics of CB amplifier. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - Draw and explain the fundamental block diagram of basic operational amplifier. | 5M | CO-4 | BL-2 |
|  | - Give the pin diagram of IC 741 and give the function of each pin. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 10 | Explain the working principle, symbol and different types of of potentiometers. | 10M | CO-4 | BL-2 |

****