| Subject Code 25ES1EC103 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

BASIC ELECTRONIC DEVICES AND CIRCUITS

(EEE)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A

5X2=10M

|  | What is a filter and mention the different types of filters used in rectifier circuits? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the relation between , and . | 2M | CO-2 | BL-1 |
|  | What are FET parameters and give the relation between them? | 2M | CO-3 | BL-2 |
|  | Brief about transistor work as an amplifier. | 2M | CO-4 | BL-1 |
|  | Define sensitivity of negative feedback amplifier. | 2M | CO-5 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - What is the effect of forward and reverse biasing of p-n junction on the depletion region? Explain with necessary diagrams. | 5M | CO-1 | BL-2 |
|  | - A Diode operating at 300 K at a forward voltage of 0.4V carries a current of 10 mA when voltage is changed to 0.42 V the current becomes thrice. Calculate the value of reverse leakage current and η for the diode. Consider VT = 26 mV. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | A full wave rectifier uses two diodes, the internal resistance of each diode may be assumed constant at 20 Ω. The transformers RMS secondary voltage from Centre tap to each end of secondary is 50 V and load resistance is 980 Ω. Evaluate: i. The mean load current ii. The RMS value of load current. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - What is the need for biasing what are the factors effecting the operating point in BJT? | 5M | CO-2 | BL-3 |
|  | - Draw the input output characteristics of NPN transistor in CE configuration and explain. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | In a Silicon transistor circuit with a fixed bias, VCC=9V, RC=3KΩ, RB=8KΩ, β = 50, VBE=0.7V. Find the operating point and Stability factor. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Describe the operation of depletion type N-MOSFET with the help of transfer and drain characteristics. | 5M | CO-3 | BL-2 |
|  | - An FET has a drain current of 4 mA. If Idss =8 mA and Vgs(off) = - 6V, find the value of Vgs and Vp. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Explain the different biasing techniques of JFET. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Brief about small signal h- parameter model of CE amplifier. Compare voltage gain, current gain, input impedance, output impedance of CE, CB and CC amplifiers. | 5M | CO-4 | BL-4 |
|  | - Draw the small-signal model of common drain JFET amplifier. Derive expressions for voltage gain and output resistance. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | The CE amplifier has the following specifications. hie=1100 ohms, hre = 2.5X10-4 , hfe=50, hoe=24µA/V if RL=10Kohms and Rs=1k ohm. Find the various gains and input impedances and output impedances | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the principle of negative feedback in amplifiers. Show quantitatively the effect of negative feedback on (i) Gain (ii) Stability (iii) Noise (iv) Distortion. | 5M | CO-5 | BL-2 |
|  | - Show that current–series negative feedback increases the input impedance and increases the output impedance. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | Starting from the description of a generalized LC Oscillator, derive the expression for frequency of Oscillation in a Colpitts Oscillator | 10M | CO-5 | BL-3 |

****