| Subject Code 25BS1CH101 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

Chemistry for Engineers

(Common to EEE, ECE, CS-AIML, CS-IOT & EVL Branches)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | List out chlorinating agents for disinfection of drinking water. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define the following terms: i) Electrode potential, ii) Ion-selective electrode | 2M | CO-2 | BL-1 |
|  | Differentiate between LPG and CNG. | 2M | CO-3 | BL-4 |
|  | Present the chemical structures of monomers used in the preparation of Buna-S. | 2M | CO-4 | BL-1 |
|  | Define self-healing materials and give an example. | 2M | CO-5 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Calculate the total hardness of a water sample with following composition: Ca(HCO3)2 = 162 mg/L, MgSO4 = 60 mg/L, Mg(HCO3)2 = 73 mg/L, CaCl2 = 222 mg/L and NaCl = 58.5 mg/Convert into oCl and oF | 5M | CO-1 | BL-3 |
|  | - Differentiate between Calgon conditioning and phosphate conditioning. | 5M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2 | - Explain the steps involved in potable water treatment. | 5M | CO-1 | BL-2 |
|  | - With the help of a neat diagram to explain the process of desalination of brackish water by reverse osmosis. | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Explain the construction of Calomel electrode with a labeled diagram. | 5M | CO-2 | BL-2 |
|  | - Discuss the sacrificial anode method for protection of ship hulls from corrosion. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Explain the rust formation by electrochemical corrosion with equations. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Explain moving bed catalytic cracking with a neat diagram. | 5M | CO-3 | BL-2 |
|  | - Draw a schematic diagram of a proton exchange membrane fuel cell and label the components. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | - Distinguish between primary and secondary batteries with an example each. | 5M | CO-3 | BL-4 |
|  | - Predict the chemical reactions involved during charging and discharging of a lithium-ion battery. | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Differentiate between thermoplastics and thermosetting plastics. | 5M | CO-4 | BL-4 |
|  | - Explain features and applications of polylactic acid. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | - Explain the classification of fiber reinforced plastics with an example for each. | 5M | CO-4 | BL-4 |
|  | - Discuss the preparation, properties and applications of Teflon. | 5M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Apply the principle of UV-visible spectroscopy in identification of pollutants in dye industry. | 5M | CO-5 | BL-3 |
|  | - Explain the working principle of STM. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | - What are shape memory alloys? Give examples with applications | 5M | CO-5 | BL-2 |
|  | - Apply the amperometric method for monitoring glucose levels. | 5M | CO-5 | BL-3 |

****